COLOR_LED  PROGRAM

Operation Manual

10 May 2007

For Picture Sign

©All Right reserved

Version 1.9x

Date: 2007-05-10

TABLE OF CONTENTS

Section 1  Introduction	4

1.1	System Requirements:	4

1.2	Using the Mouse and Keyboard	4

1.3	Navigating the Color_Led V1.9x Software	5

1.4	Basic Steps to Use the Color_Led V1.9x Software	6

Section 2  Installation & Configuring the Software	7

2.1	Installations of Color_LedV1.9x	7

2.2	Configuring the Color_Led 1.9x	7

2.2.1	Set communication mode	7

Section 3 Color_Led V1.9x Menu Introduction	9

3.1	Main Menu introduction	9

3.3.1	File Menu	9

3.1.2	Program Menu	10

3.1.3	Timing menu	10

3.1.4	Call Menu	10

3.1.5	Send Menu	10

3.1.6	Set Menu	11

3.1.7	View menu	11

3.1.8	Language menu	12

3.1.9	Help menu	12

3.2	New File Types	12

3.2.1	Creating Program File	12

3.2.2	Creating Time File	13

3.2.3	Creating Timing file	15

3.2.4	Creating Week Timing File	16

3.2.5	Create ZText File	17

3.2.6	Creating ZRT File	18

3.2.7	Creating Picture File (BMP, JPG, PNG)	19

3.2.8	Creating Call List File	19

3.2.9	Creating Temperature File	20

Section 4  Operation Guide (Operation Example)	21

4.1	Creating a New File	21

4.1.1	Creating New Message	21

4.1.2	Creating a Time File	22

4.1.3	Creating a Picture File	23

4.1.4	Creating a Program File	23

4.2	Opening an Existing Program File	25

4.3	Creating the Schedule(s)	25

4.3.1	Starting Hour Timing	25

4.3.2	Starting Week Timing	26

4.4	Running the File/Schedule on the Display	26

Appendix	27

I.	Attributes of AVI File	27

II.	Attributes of Text & Picture File	27

III.	Attributes of Time & Temperature File	28

IV.	Special Functions	29

Statement	31

# Section 1  Introduction

The purpose of this manual is to assist in the installation and operation of the Color_Led V1.9x.

## System Requirements:

The COLOR_LED V1.9x has the following minimum system requirements:

  - • Personal computer running Windows 98, Me™, NT 4.0, 2000 or XP Home/Professional with the         most current updates installed.

  - • Personal computer using a Pentium4 200 class or higher microprocessor is recommended.

  - • 128MB RAM (256MB RAM is recommended).

  - • CDROM Drive.

  - • Mouse or other pointing device.

  - • One Color_Led V1.9x installed.

  - • 100MB hard disk space

  - • With display mode of 800x600, 16, 24 or 32 bit colors

  - • 10M Network

## Using the Mouse and Keyboard

The following terms are used to describe various mouse actions. These instructions are based on the standard, right-handed mouse configuration, with the left button acting as the primary mouse button and the right acting as the secondary mouse button.

  - • CLICK means quickly click and release the primary mouse button.

  - • DRAG means to click and hold the left mouse button when the cursor is on an item. Then move the cursor while holding the left mouse button down and the object will move with it.

  - • DOUBLE-CLICK means quickly press and release the primary mouse button twice.

  - • RIGHT-CLICK means click the button on the right side of the mouse.

  - • HIGHLIGHT means to click the mouse button over text. The text will have a colored bar placed over it.

  - • SELECT means to click the mouse button over text. The text will be marked, such as by a check mark T, in a nearby box.

An uppercase word in brackets (such as [OK]) represents a button on the screen. The names or letters of keys on the keyboard will be given enclosed in arrows (such as <ENTER>) to differentiate between the screen commands. For two or more key combinations such as <ALT+E>, <CTRL+HOME> or <CTRL+SHIFT+SPACE>, hold down the first key (and the second key, if applicable) as the last key is pressed.

Many of the options in the Color_Led V1.9x software application can be accessed with the keyboard as well as the mouse. Where applicable, both mouse and keyboard actions are given to accomplish a command. Many screen buttons have labels which have a letter underlined. Pressing the key on the keyboard that correlates with the underlined letter will activate that particular button. To activate secondary buttons, press the underlined key while holding the <SHIFT> key. To activate menu bar items from the keyboard, press the underlined key while holding <ALT>.

## Navigating the Color_Led V1.9x Software

The software is user-friendly. When the software program starts, a “main window” loads. Each function of the software is launched from this program. Click on the appropriate menu or button to activate the desired function.

Note: A new files Window (Program file, Time file, Text file, Picture file etc.) will appear only if the new File is clicked and activated on Color_Led software.

The main window includes Title bar, Menu bar, Tool bar & Status bar. In the following, we will give a simple introduction about them.

Title Bar

Toolbar

Menu Bar

Status bar

[Fig.1.1 Software Interface]

Title Bar

Title bar shows the current title information.

Tool Bar

Tool bar provides user the shortcut of menus.

Menu Bar

Main menus on the Menu bar: file menu, timing menu, program menu, call menu, send menu, set menu, view menu, language menu, help menu. Please click to choose the submenu from the pull-down list.

Status Bar

Status bar shows additional information such as screen, ready etc.

## Basic Steps to Use the Color_Led V1.9x Software

This manual is laid out in basic order to create and display a message. In summary, the steps you will need to follow are:

1. Install the Color_Led software if it is not already installed on your computer (Section 2).

2. Configure the display type(s) and communication method (Sections 2).

3. Create the message(s) (Section 4).

4. Create the schedule(s), when desired (Section 4).

5. Run the message/schedule (Section 4).

This manual will walk you through the steps necessary to help you use your display system to its full potential.

# Section 2  Installation & Configuring the Software

## Installations of Color_LedV1.9x

The Color_Led 1.9x software must be installed to a hard disk before use. To install the software onto a computer, follow these steps:

1. Place the Color_Led 1.9x installation compact disk (CD) into the appropriate CD ROM

(Presume drive G:).

2. The installation should begin automatically within a few seconds. If it does not, click

on the Start button and select Run from the menu. Type “G:\” and click the setup program to install it.

3. Follow the instructions on the screen. Setup will copy the files necessary to run the

Color_Led 1.9x software to the hard disk and create Color_Led 1.9x in the Start menu.

Note: The Color_Led 1.9x software installs to the C:\Program Files\ Color_Led 1.9x directory by default; this is the recommended location.

4. The Color_Led 1.9x software is now installed and ready to be configured for using

with the Color_Led 1.9x Display Network.

## Configuring the Color_Led 1.9x

Run the program by double-click on the COLOR_LED V1.9x icon on the desktop. Left-click Set | LED Set (pull-down list) on the top tool bar of the program, then a dialog would pop-up like.

### Set communication mode

There are three kinds of communication modes—Serial Communication (COM Port), LAN & Modem.

Select “LED Set” option under “Set” menu. It will display a dialog as [fig2.1 LED set].

Serial communication
Please refer to the following steps to setup it:

Firstly, please select COM port on your PC that connects with LED display by cable in box【1】. One end of cable is connecting to the control PC, another end is connecting to LED display. This cable transmits messages to each LED display.
Secondly, select communication baud rate when transmitting messages in list box【2】. As usual, the baud rate setting is 38400bps. But user should choose baud rate according to the LED display. 
Thirdly, input the address of target LED display which you want to send. 
Fourthly, set resolutions (width + height, unit is pixel) for LED display, length of communication frame, input the flash disk size as well as click Save LED file & then click OK to quit.

LAN (Local Area Network) communication 
As a must, the network of LED display and personal computer are linked into the same local network.

For instance, the IP setting of the LED display is 192.168.0.239. Then IP setting for controlling PC has to be 192.168.0.xxx. The xxx means that the user can set any number for the controlling PC.  If it responses, that means the IP setting for communication is ok. If pop-up a request says timed out, then it is failed to set the IP. The user needs to set it over again. The TCP/IP allows the possibility to connect one or more signs to a Local Area Network (LAN) using Ethernet Adapters as well as controlled by PC in the same network. Before transmit messages to Signs by LAN, the user has to configure Ethernet Adapter correctly. 
Please refer to the following steps to setup it: 
Firstly, select ‘LAN’ in box【1】.
Secondly, input the IP address of the LED display. 
Thirdly, set resolutions (pixels) for LED display, select flash disk size as well as click Save LED file & then click OK to quit.
IP Address – It means the 4-node Internet Protocol address for the Ethernet Adapter connected to Sign. Please ask your network administrator if you don’t know this address.

MODEM communication
Modem (or "remote") connection- Typically, this method is taken when the sign(s) you want to transmit messages is (are) far away from the controlling PC. For this way, a modem is connected to your PC and another modem is installed by side of one or more signs at the far distance. Messages are transmitted to the signs by modems (the PC modem would call the Sign Modem to send the information). 
Please refer to the following steps to setup it:
Firstly, select ‘MODEM’ in box【1】.
Secondly, set baud rate for the modem.
Thirdly, input the address for LED display. 
Fourthly, set resolutions (pixels) for LED display, length of communication frame, select flash disk size     as well as click Save LED file & then click OK to quit.

[Fig2.1 LED Set ]

【1】

【2】

【3】

【4】

【5】

【6】

【7】

【8】

# Section 3 Color_Led V1.9x Menu Introduction

## Main Menu introduction

### File Menu

Click the File menu, it has a pull-down submenu, select one and Click to run it.

File | New- To create a New file, from this button the user can create files as follows: Program File, Time File, Picture File (BMP, JPG, PNG), Timing File, Week Timing File, ZRT File, ZText File, Call List File, Temperature File. Shortcut: Ctrl+N

Open- To open an existing file that stored in the PC. Shortcut: Ctrl+O

Save- To save an editing file or new file to the PC.  Shortcut: Ctrl+S

Save as- To save a file as another name or to another directory. Shortcut: Ctrl+A

Quit- To close the Color_Led V1.9x.  Shortcut: X

New File types:

    - Timing File
To schedule a task according to the hours/minutes/seconds when desired. Through this function, the display can show the contents timely as per user’s schedule. The file format is .AT.

    - WEEK Timing File
To schedule a task according to the week when desired. The only difference is that the time is week. If user wants a file displayed at specified time in a week, it needs a week timing file. The format is .WT.

    - Time file
Time file contains time, date, week and year. If you want display time, date and week on LED display, you can create a time file. The format is .TIM.

    - ZText File
It is a text file. ZText file don’t need others picture files or timing files as motherboard, it may display clearly. The format is .ZTX.

    - ZRT File
Just like a text file, but it has to be sent as picture format. The format is .ZRT.

    - Program File
It is a general file program. All other files such as Timing file, Picture file, Html file and Time file must be added into a Program file as ZPF file while edited, then run it. Or those other files would be ran by the program and be displayed on the display. The format is .ZPF.

    - Picture File
To create pictures. The picture file provides three formats: .BMP, .JPG, and .PNG.

    - Temperatures file
To create a temperature file when you want display temperate on LED display. The format is .TEM.

    - Call list file
When you want to display many files on display, you may create a call list file. The format is .CAL.

To open a new file (whatever file types), please highlight the file type and name it, click OK, the editing dialog box appears, and then edits it on the main window. (Details please refer to 3.2)

### Program Menu

Click the Program menu, it has a pull-down submenu, select one and Click to run it.

Program | Region List- It will activated and show the current region lists for editing file, from which you can select the region number and edit it.

Add Region- To add a Region, 10 regions at maximum for a program file. (0~9)

Del Region- To delete a region.

Edit Region- To edit the current region.

Preview- To preview a program file.

Note: This menu is enabled only upon an opening of the program file.

### Timing menu

Click the Timing menu, it has a pull-down submenu, select one and Click to run it.

Timing | Preview- To have a preview on the effects on the screen, it is activated & available only for a timing file.

Close- To close the timing file editing box, it is useless for other file types but only the timing editing box.

### Call Menu

Click Call menu, it has only one pull-down submenu Close.

Call | Close- To close the call list viewing box.

### Send Menu

Click the Send menu, it has a pull-down submenu, select one and Click to open it.

Send | Send File- To send a program file to the display.

Send Real Msg- To send a Real Time Message to the display; When send a real msg, the messages which is running on the display will be stopped and the real msg will be displayed, after finish the running, the screen will turn to normal display with original messages.

Get File List- Get the file list information of LED display.

Get Time- Get the time information of LED display.

Adjust Time- To checkout and adjust the time to be the same with controlling PC for Led Display.

Send Reboot- To send a command to reboot the display screen.

Update LED OS- To send a command to update firmware for LED Display.

Resume LED Disk- Resume the disk for LED Display.

Note: The LED display must be connected to the computer and turn on power before execute any command of ‘Send’ menu.

### Set Menu

Click the Set menu, it has a pull-down submenu, select one and Click to open it.

Set | LED Set- To setup the communication mode and LED Display’s size. (Please refer to 2.2)

Select Modem- To communicate through Modem device. Click pull-down list button, the Select Modem dialog box appears; please refer to the following [Fig. 3.1].

[Fig.3.1 Select Modem ]

Installed modem(s) – From the pull-down list, it will list all modems installed in your PC. If PC has installed more than one modem, user must select one modem as the defaulted device to transmit messages to Sign.

Port – Input the COM port of PC that connected to modem.

After set above parameter, please click OK button to save and quit.

Dial/hang up- When transmit messages to Sign through modem, connect PC to Sign at first, and then Click Dial/hang up to connect.

When sending over is checked, it will hang up the connection automatically when messages are transmitted over.

### View menu

Click the View menu, select and click to pen the submenu from the pull-down list.

View | Toolbar- Click to select the toolbar. If selected, the mark “√”appears before the menu name. If not, then the toolbar will be hidden.

Status Bar- The same like Toolbar.

### Language menu

Click the Language menu, select the language from the submenu pull-down list. The Color-Led V1.9x provides three kinds of language for operation interface. Chinese, English & Japanese.

### Help menu

Click Help menu, select the About Color_LED, the Software version information appears. Or click the shortcut icon  to view the version information.

## New File Types

### Creating Program File

1: Click ‘New’ menu in the File pull-down list or shortcut key  on toolbar. It will display a dialog box named ‘New’.

2: Input file name, select file save location and select program file in following fig. then click button OK, the editing window appears.

[Fig.3.2 New]

[Fig.3.3 Program File]

3: Right-Click in the blank area to select ‘Add Region’ in the pop-up list or click  button to add a region in black area. 10 regions at Max.

4: Put the mouse at the region frame and drag it to edit the size. Select the region and Right-click to add a file, please refer to following [Fig.3.4 Adding Files]

[Fig.3.4 Adding Files]

5: Select the file type and then click to add it.

6: Set attribute for file or function by clicking on corresponding attributes column.

[Fig.3.5 Attributes]

7: Double-Click the file name in type column to open the file, then you can edit content of this file.

Or highlight the file name in type column then click “Delete” key on keyboard to delete the file.

8: Click ‘Preview’ under program menu or  button to preview the running effects.

### Creating Time File

1: From the New dialog [Fig. 3.2 New] highlight the Time file name then input the file name, select the 

save location, then click OK , the editing window appears.

2: Select the time/date/week from the pull-down list that showed in dialog [Fig.3.6]. Then input the value for X coordinate in box:  and value for Y coordinate in box: , for the X/Y coordinates that means the coordinates for the time/date/week showed on the screen.

[Fig.3.6 Time file]

Set time: In above dialog, click button  to set the time. It will display a dialog box as following:

[Fig.3.7 Date/Time Properties]

In above dialog, there are three options:

- : This option is to setup time and date.

- : This option is to setup the time zone. In fig2.22, it shows this option. User may select

the time zone in list box.

- : This option is to adjust time to be the same with internet time.

### Creating Timing file

1: From the New dialog  [Fig. 3.2 New], highlight the Timing file name then input the file name, select the save location, click OK, the editing window appears.

2: From the editing window, right-click in blank area, select “Add timing” in pop-up list or click  button to add a schedule.

[Fig.3.8 Timing file]

Set Hour

Set minute

[Fig.3.9 Timing ]

3: Highlight the time then right-click it to select the “Add a program” in the pop-up list or click  button to add a program file. The OPEN dialog box will appear, select a program file and click OPEN.

[Fig.3.10 Timing ]

Highlight it

4: Select Timing | Preview menu to preview the effects.

5: Select Timing | Close menu to close the editing window.

### Creating Week Timing File

1: From the New dialog  [Fig. 3.2 New], highlight the Week Timing file name then input the file name, select the save location, click OK, the editing window appears.

2: Highlight the week date by clicking it, then click button  to add a Timing file. It is similar to add a program when create timing file.

[Fig.3.11 Adding a timing file]

### Create ZText File

1: From the New dialog  [Fig. 3.2 New], highlight the ZText file name then input the file name, select the save location, click OK, the editing window appears.

2: Input the text in black area of the editing window.

3: Select the font in list box. The default font is “ZFONT 14”.

4: Select the color for font by clicking.

6: Save your file and close the window.

[Fig 3.12 Ztext file]

Note: If your text is chinese characters, you can’t change the font. It is a standard 1616 lattice text that will display chinese character on 1616 display screen. Only it is english characters, you may change the font by select font in list box.

### Creating ZRT File

1: From the New dialog  [Fig. 3.2 New], highlight the ZRT file name then input the file name, select the save location, click OK, the editing window appears.

2: Input the text in the editing area.

[Fig.3.13 ZRT files]

3: Click  to setup the font. Select the font type from the pop-up font list.

4: Save file and close the window.

[Fig.3.14 Font]

### Creating Picture File (BMP, JPG, PNG)

The Color_Led V1.9x provides three formats of pictures: .BMP, .JPG, and .PNG. But, it takes the same steps to create all kinds of pictures, so please refer to the following steps.

1: From the New dialog  [Fig. 3.2 New], highlight the Picture file name then input the file name, select the save location, click OK, the editing window appears

[Fig 3.15 Paint picture]

2: Drawing a picture or open an existing picture file from the PC and edit it.

3: Save the file & close the window.

### Creating Call List File

1: From the New dialog  [Fig. 3.2 New], highlight the Call List file name then input the file name, select the save location, click OK, the editing window appears.

[Fig.3.16 Call list]

2: In above dialog box, right-click on white area, from the pop-up menu list, follows are available:

Select “Add program” command in the menu or click  button to add a program that has saved on disk. Select “Del program” command in the menu or click  button to delete a program from call list.

Select “Move up program” or “Move down program” command in menu or click  /  button to change the order of program in call list.

3: Save the Call List and close the window.

Note: After send call list file to LED display, user can select a program file that included in call list file, then select “Call program” command in the right-key menu or click  button to call it. And the selected program file will be displayed on LED display.

### Creating Temperature File

1: From the New dialog  [Fig 3.2 New], highlight the Temperature file name then input the file name, select the save location, click OK, the editing window appears.

[Fig.3.17 Temperature file]

2: From the box             select the option that you want to set, then input the value for X / Y

coordinate in coordinate box.

3: Save it and close the window.

# Section 4  Operation Guide (Operation Example)

All Menu functions and operations please refer to Section 3.

## Creating a New File

For example, here we suppose the user want to send a picture + text + time/date like below showed to the display. We suppose the file name is Example.ZPE.

[Fig 4.1]

So, firstly, we have to create the Messages, Time & Picture separately, then buildup three files to a Program File to send.

To create this program file, there are following steps mainly:

Step 1- Create the Text file;

Step 2- Create the Time file;

Step 3- Create the Picture file;

Step 4- Create the Program file;

### Creating New Message

- Click on the new button  or select New from the File pull-down menu. Please refer to the following New dialog.

[Fig 4.2]

[Fig 4.3]

Please select Ztext File to create a new text message, name it as Text and select the defaulted directory then click OK.

Input the message in the editing window, select the font & Color then click  button.

[Fig 4.4]

Creating a Time File

- From the dialog [Fig.4.2] please select a Time File and name it as Text, and select the defaulted directory, then click OK.

Setup the coordinates for each item respectively through the value of X and Y, or put the mouse at the top left corner of the item then drag it directly to the location you want.

Click  button to save it to the defaulted directory.

### Creating a Picture File

- From the dialog [Fig.4.2] please select a Picture File(jpg format) and name it as Picture File, and select the defaulted directory then click OK.

Please create a new or open an existed picture through the DRAWING software.

After you finish it, please save it.

### Creating a Program File

- From the dialog [Fig. 3.2] please select Program to build a new program file, name it as Example.ZPE, click OK and then there would pop-up an editing window like [Fig.4.5].

[Fig 4.5]

Adding the Region- Right-Click the mouse on the blank area, add a region first. (please refer to [Fig.4.5])

Here we add three regions (region 0, 1 & 2), then edit them respectively as follows introduced.

Editing the Region- The user can edit the region through Right-Click the mouse | Choose Edit Region, or put the mouse at the frame and drag it to change the region size.

[Fig 4.6]

So, setup the first region to X=0, Y=0, W=91, H=75 (or other size you like);

The second region to X=0, Y=74, W=91, H=54 (or other size you like);

The third region to X=90, Y=0, W=166, H=128 (or other size you like);

So, the windows appearance will be [Fig.4.7]

[Fig 4.7]

Adding the file- Right-Click on the Region area 0 to choose Add Text, open the Text.ZTX file, setup the attributes for the Text.

Attribute 2= Move Top<-Bottom,     Attribute 3= Move clear Top<-Bottom;

Attribute 4= Speed 1,               Attribute 5= Pause 0 second

[Fig 4.8]

- Right-Click on the Region area 1 to select Add TIME file, open the Time. TIM file.

- Right-Click on the Region area 2 to select Add Picture file, open the Picture. JPG, like the first step, to setup its attributes.

Attribute 2= Move Left<-Right,    Attribute 3= Move clear Left->Right;

Attribute 4= Speed 1,            Attribute 5= Pause 0 second

At the last, click  to view the effects and then close the window, the program will save it automatically and quit the current editing window. (About the Send, please refer to 3.4)

## Opening an Existing Program File

To open an existing program file for editing or viewing:

1. From the File pull-down menu, select, or click. The Open dialog box appears.

2. Select the file type and the file directory.

3. Highlight the schedule name and click OPEN.

## Creating the Schedule(s)

There are two ways to schedule the task, they are:

- Hour Timing

- Week Timing

### Starting Hour Timing

1. From the File pull-down menu, select New, or click. The New dialog box appears.

2. Select the Timing File, name it and click OK, the Timing dialog box appears.

3. Right-Click on the blank area and select Add Timing.

4. Highlight & Right-Click on the Time, select Add Program, and the Open Dialog box appears.

5. Highlight the program name and click OPEN.

6. After you finish the timing, click Save and Close in Timing pull-down menu to quit.

[Fig 4.9]

### Starting Week Timing

1. From the File pull-down menu, select New, or click . The New dialog box appears.

2. Select the Week Timing File, name it and click OK, the Week Timing dialog box appears.

3. Highlight the date and then click, the Open dialog box appears.

4. Highlight Hour Timing file and click OPEN.

5. When finish all the schedule, click Save and  to quit.

## Running the File/Schedule on the Display

To send a message from the Color_Led V1.9x controller computer to a display, click Send | Send File, highlight the name of the file in the Open dialog box. Click OPEN. The file will be opened and sent to the display.

Before we send file to the sign, we can open it first and then click  to take a preview on the effects.

# Appendix

## Attributes of AVI File

AVI file is a kind of video file. Through Color_Led software, the AVI file is able to be added into the program file and to be sent. It is best to be less than 500k for an AVI file. Usually, it needs to be made by user.

When display a video, it is necessary to setup its attributes. For example, when a part of the video needs to be displayed, then you may select to display from frame 1 to frame 200, the number of frame is defined by the user. However, for most of the small AVI files, there is no necessary to make it many frames.

Please refer to dialog box [Fig. 5.0], we can setup its attributes.

[Fig. 5.0]

- Type: The file type such as VIDEO, TEXT and TIME etc.

- Attribute1: The path of file.

- Attribute2: The frame number to start.

- Attribute3: The frame number to stop.

- Attribute4: Moving speeds. There are eight kinds speeds as SPEED 1, 2, 3… SPEED 8. The speed is listed from fast to slow, i.e. SPEED 1 is the fastest and SPEED 8 is the slowest.

- Attribute5: Display area size. There are four modes: 50, 100, 200 and Custom. 
“50” means 50%, reduce a half size.
“100” means 100%, it is normal size.
“200” means 200%, zoom in twice size.
“Custom” means defined size by user.

## Attributes of Text & Picture File

Text file formats: Word file (.DOC), ZText file (.ZTX) and ZRT file(.ZRT).

Picture file formats:.JPG, .BMP and .PNG.

All these files should be added into a program first then setup the attributes. Please refer to the below dialog box (steps for all these files are the same).

[Fig. 5.1]

- Type: The file type such as VIDEO, TEXT, TIME and so on.

- Attribute1: The path of a file.

- Attribute2: The moving method. 14 modes in total. 
Null: no display.
Show: displaying in static status.
Move left->right: moving from left to right.
Move left<-right: moving from right to left.
Move top->bottom: moving from top to bottom.
Move top<-bottom: moving from bottom to top.
Open top<-middle->bottom: opening from middle to both top and bottom.
Open left->right: opening form left to right.
Open left<-right: opening from right to left.
Open top->bottom: opening from top to bottom.
Open top<-bottom: opening from bottom to top.
Color: changing gradually the brightness from dim to bright.
Flash: flickering regularly.
Random: displaying at random.

- Attribute3: The exiting method. 14 modes in total. All these modes are the same as defined in      attribute 2.

- Attribute4: The moving speeds. 8 speeds: SPEED 1, SPEED 2,…, SPEED 8. Arranged from high to low, i.e. SPEED 1 is the fast speed and SPEED 8 is the slowest speed.

- Attribute5: The pause time. The unit is tenth of a second.

For instance, “1” means 0.1 second, “10” means 1 second, “100” means 10 seconds.

## Attributes of Time & Temperature File

The time file or temperature file should be also added into program too, and their attributes setting are the same. Please refer to the following dialog box.

[Fig. 5.2]

- Type: The file type such as VIDEO, TEXT, TIME and so on.

- Attribute1: The path of a file.

- Attribute2: The pause time. The unit is tenth of a second. For instance, “1” means 0.1 second, “10” means 1 second, “100” means 10 seconds.

- Attribute3: Disabled.

- Attribute4: Disabled.

- Attribute5: Disabled.

## Special Functions

When create a program file, there are some special functions may be taken. The following [Fig.5.3] shows the right-key menu that add files or function to a program file.

[Fig.5.3]

- Loop Function
When a file needs to be displayed repeatedly, then the Loop function can meet the need. Through Loop function, a circle sentence could be set up. Right-click in the region area then select “Add Loop Head” command (it is where the file starts) and then click “Add Loop End” (it is where the file stops) command. After it displays in active area “Loop” and “End”, highlight the file name that you want to display repeatedly and right-click it, it will show a pop-up list “Up” and “Down”. Clicking “Up” and “Down” command to move it to be between “Loop” and “End”. In “attribute1” to select the repeating times. Now, file(s) between “Loop” and “End” will be displayed circularly. Please refer to the 

following Fig. for this function.

[Fig.5.4]

- Signal & Wait Function

When sending a series of files that has sequential display, for example, to display these orderly: main title, subhead, chapter and section or special display and so on, then it is necessary to use “Add Signal” and “Add Wait”. If only “Add Signal” selected, but doesn't have “Add Wait” command, the “Add Signal” would not react. Its function is to control contents in different regions. 
For example, we assume that there are two regions in a program. “Region 0” has two files, we suppose them “A” and “B”, and it needs 10 seconds to run over all the contents in region “0”; “Region 1” has five files, we suppose them “1”, “2”, “3”, “4” and “5”, and it needs 60 seconds to run over all the contents in “Region 1”; We now want “region 0” display content “A”, while “region 1” display content “1”, “2”, “3”, then “region 0” displays content “B”, “region 1” displays content “4”, “5”. If we don’t set “Wait” and “Signal”, the process is that “region 0” displays repeatedly until “region 1” runs over. Therefore, we need to set “wait” and “signal”. User must add “Add Wait” between content “A” and “B” in “region 0”, and add “Add Signal” between content “3” and “4”. 
NOTE: In “attribute2” can set overtime for wait.

# Statement

Dear Customers:

Thank you for purchasing this product！

For optimum performance and safety, please read these instructions carefully.

Before operating or any use of this product, please read these instructions completely and keep the manual for future reference.

Changes

The manual we provide ‘as is’ is without any warranty, expressed or implied, including but not limited to the implied warranties or merchantability and fitness for a particular purpose. We may make improvements and (or) changes to the program(s) described in this manual at any time without notice.

This manual may contain technical inaccuracies or typographical errors. Changes are periodically made to the information in this manual; these changes are incorporated in new editions of this manual.

Warning

When using this software for non-our company’s equipment, the reliability generally would not be responsible.

The figure interfaces in this manual are for a purpose only of guiding users while uses the program. Because all figures are cutting figures, they may not show the content of interface clearly.

Copyright

All rights reserved. Part of this document must not be copied, reproduced or translated. It shall not otherwise be recorded, transmitted or stored in a retrieval system without the prior written consent of our company.

If any content of this manual has any discrepancy with software of product that bought by the user, our company retains the final explanation rights.