MANUAL

LED ELECTRONIC DISPLAY

USER MANUAL OF OUTDOOR PICTURE SIGN

PICTURE SIGN

User Manual of Pictuer Sign

Outline

The Picture Sign is one of our late developed products. Different from the Video Display that is always connected with a PC, the Picture Sign works asynchronously with the control PC.

With powerful embedded control system (ARM-based) and embedded operation software (Linux-Based), it can display not only texts, but also colorful pictures, flash files, short video (avi) files etc. All displayed content are stored in on-board memory and can be updated through TCP/IP or RS232/422 or Modem connection etc. Network connection allows a group of Picture Signs to be controlled from a central control point.

It is available and affordable in form of single color, RG color and full color, and can be indoor, semi-outdoor(window brightness) and outdoor use with different sizes and pitches.

Software that runs on the control PC is powerful windows-based and easy-to-use, and allows the creation, scheduling and display of content on the Picture Sign. The software also gives the user the flexibility to control content in an instant.

Features

- Supporting file types: JPG, PNG, BMP, DOC,TIM,TIMING,ZTX, S WF, AVI, etc.

- Supporting real-time insert broadcasting.

- Supporting information updating.

- Maximum Regions: 10

- Volume of 1000 programs and 1,000 pictures.

- Clock and 288 timers contained.

- System time adjustment.

- 14 way s to display.

- 8 kinds of moving speeds, 0~~99999 seconds of time out.

- Time lists available.

- Program contents will be saved in LED screen and run automatically .

- Color & Shades: Monochrome, RG, or RGB. Each color and each pixel can 
contain 256 shades.

- Maximum resolution: 256x256 pixels.

- Thank you very much that you purchase our product

After opens the packing box, please first inspects the fitting according to the detailed list of supply items. If there are something has damaged or lack, please contact your dealer as soon as possible!

Catalog

0.	Foreword	5

1.	Introduction	7

1.1	System Outline	7

1.2	Element Identification	9

1.3	Software	11

2.	Mechanical Installation	13

2.1	Overview	13

2.2	Cabinet’s information	14

2.3	Element Assembly	17

2.4	Installation method	18

3.	Electrical Connection	21

3.1	Cable and common connector	21

3.2	Circuit Diagram whole System	22

3.3	Signal connection	24

3.4	Power Connection	27

3.5	Fans Connection	29

4.	Maintenance &Troubleshooting	30

4.1	Maintenance	30

4.2	Troubleshooting	31

4.3	Return and Repair	32

5.	Appendix	33

A. Some things referenced within the manual	33

B. File Log	34

6.	Statement	35

- Foreword

The foreword partial has mainly outlined the User Manual of picture sign. Mainly includes:

- About this Manual

- Information feedback

About This Manual

This Manual is about picture sign. It explains the electronic connection, installation, maintenance and troubleshooting of the display.

Reader of the Manual

This manual is for the user and the Maintenance of equipment personnel as well as who needs to purchase our product to write. Hope it will bring some help for you!

Contents of the manual

This manual includes six parts: Introduction, Mechanical Installation, Electrical Connection, Maintenance and Troubleshooting, Appendix, Statement.

- Introduction: Description the basic information needed to use the system of outdoor display. Take time to read the entire introduction as it defines terms and explains concepts used through the manual.

- Mechanical Installation: gives guidance on Display assembling and mounting.

- Electrical Connection: gives guidance on terminating power and signal cable of the Display.

- Maintenance and Troubleshooting: Describe such things as replacing basic       display components or modules, troubleshooting the Display, performing general maintenance.

- Appendix: List the software, manuals referenced within this manual.

- Statement: States the copyright and others.

Safety Precautions

  - Read and understand these instructions.

  - Personnel must be professional to installing and maintenance.

  - Display systems must be connected to earth-ground. We recommend a resistance to ground less than 10 ohms

  - Avoid locations that are subject to direct sunlight, excessive heat, moisture and dust.

  - Don’t modify the Display structure or attach any panels or coverings to the Display.

  - Unplug this product from the wall outlet before cleaning. Do not use liquid cleaning agent or aerosol cleaning agent. Use a damp cloth for cleaning.

  - This product should be operated from the type of power source indicated on the marking label. If you are not sure of power available, consult your dealer or local power company.

  - Disconnect power when servicing the display

  - Please inspect the fixed equipment when you install the lager display.

- Introduction

This part has introduced some elementary knowledge and technical specification of the system. It including following content:

- System outline

- Element identification

- Software

  - System Outline

- System structure

This system is a picture sign for outdoor use. It adopts modularized design. The modularized design allows easy installation and maintenance. Display area is  meters high by  meters width, total resolution is 80 pixels high by 96 pixels wide. It is double-face wall display. And made up a cabinet that can open at two side.

- Pixel Configuration

In this system, the pixels are made of two red LED lamps. The pitch is 12mm.

- LED cluster module

In this system, the LED cluster module is 8(h)16(w) LED pixels. There are louvers on LED cluster modules to increase the level of contrast on the Displays surface and direct LED light.

- Technical Specification

Table1 list the technical specification of this the Full-color Video LED Display.

Table1 Technical Specification

| Item | Technical Parameters |
| --- | --- |
| Pitch | Pitch 12mm |
| Pixel Configuration | 2R |
| Display Area | 0.960mm1.152mm |
| Display Resolution | 80(H)96(W) |
| Viewing Angle of LED | Horizontal: 110o, Vertical: 55o |
| LED Driving Mode | 1/4 Constant Current |
| Maximum power | 7.6KW |
| Power Input | 110V, 50Hz |
| Communication Mode | RS232, TCP/IP network |
| Continuous Working Time | Unlimited |
| Lifetime of LEDs | > 100,000 Hours |
| Working Environment | Temperature: -10 oC～+40 oC; Humidity: 15%～95%RH |

  - Element Identification

The following illustrations depict some of the more commonly accessed elements of the LED Displays.

- LED Cluster Module: An aggregation of some LED clusters/pixels and driver circuits.

- Control board (CST-HD-01M):  The control board installed in LED Screen receives signal from control computer through communication cable and process them.

- Scan board (DEMO-02C): It installed on control board and distributes the data to LED modules that control board has received.

- Light sensor board (CST-LT-CB-01): Received the environment’s brightness signal from light sensor, and process them.

- Light sensor: Used for Automatic Brightness Adjustment.

- Power Supply: Converts AC line voltage from the load centre to low DC voltage for control boards and LED cluster modules. All the power supplies are switching power supply.

- TCP/IP Adapter: Used for TCP/IP communication. It changed the TCP/IP signal to RS232 signal.

Fig1.2 Scan board

Fig1.1 Control board

Fig1.3 TCP/IP adapter

Fig1.4 Light sensor board

Fig1.5 Power Supply

Fig1.6 Light sensor

  - Software

This system has a management software: COLOR-LED

Following will introduce COLOR-LED software simply. More information please refer to user manual of COLOR-LED software.

  - Introduction

The color LED program is a very powerful program. It is designed to give the most detailed full color or gray scale graphics possible. If your desire is great graphics and you are knowledgeable in graphic programming, this is the program you want. The only limitation of this program is the skill of the programmer. Basically what this program does is turns the message board into a monitor. The program does not make the image; the program only translates your image and displays it on the message board. Because this program is designed for graphics we do not recommend a message board less than 160120 pixels. The pixel grid should be designed to match standard video grids such as 160120, 320240, and 640480. Sizes other than these will require you to crop your graphics and videos.

There are many programs you can use to make your graphics. Any graphic with an extension of jpg, png, bmp, swf, or .doc can be displayed. You can also display videos in avi format. Graphic programming software is not included.

There are 12 methods of animated movement for opening and closing these graphics. The basic function of this program is to display individual images therefore scrolling long messages left to right is not compatible with this system. If scrolling is required you must program the scrolling message as a video such as the introduction of a web page in a swf or avi format.

  - Running environment

Minimum PC System Requirements:

OS: WIN9X/2K/XP and up version.

Processor: Pentium200 and up.

RAM: 128MB and up.

Display memory: 32MB and up.

Hard Disk: 100MB and up hard disk space.

Display mode: designed for 800*600,16, 24 or 32 bit colors.

Network: 10M and up TCP/IP port.

Fig1.7 LED Set dialog box

【1】

【2】

【3】

【4】

【5】

【6】

Installing and setting
Installing software:
If you want to use a software, you must install the software on computer. 
Insert the CD labeled “Color LED” into your CD disk drive. Double click the icon of a computer monitor named COLOR_LED. Follow the instructions on the screen to install the software. A shortcut will be made on your desktop with the same icon of the computer monitor.
Setting software:
Open the program by double clicking on the COLOR_LED icon on the desktop. On the top tool bar of the program left click on “Set” and then “LED Set”. And a dialog box will be displayed as fig1.7.
In the list box【1】select the communication mode: LAN(Local Area Network), COM(Serial port), MODEM. If you use COM or MODEM, must set the bps in list box【2】. The default bps is 38400. and set the address in text box【3】. If you use LAN, must set the IP address in text box【4】. The IP address is an address that specify the 4-node Internet Protocol address for the Ethernet Adapter connected to Sign to which you will talk. Ask your network administrator if you don’t know this address. User must remark each node of IP address must less 255. Then enter the LED width and height in text box【5】and【6】. The unit is Pixel. A pixel is several LEDs that are connected together in a group. You can find these numbers on the label of the COLOR_LED CD. The Save LED file box should be checked. After set the parameter, click OK button to confirm.You are now ready to start programming.

- Mechanical Installation

  - Overview

Because every installation site is unique, there is no standard method of mounting. As manufacture of LED display, we don’t supply support structure, we can only provide recommending for the support structure design. It is customer’s responsibility to ensure that the structure and mounting hardware are able of supporting the Screen, and according with local laws.

Support structure design depends on the mounting methods, screen size and its weight. The structure design is critical and should be done only by a qualified individual and organization.

The following must be considered first before installation:

- Design support structure and groundwork; calculate carefully the needed strength according to local codes; especially note the wind load and the issue of Seismic Zone.

- The mounting structure will provide a straight and square-mounting framework for the Screen.

- For the reason of heat dissipation, the waterproof of the back of LED module is IP45; consider if any added shield needed for more protection. If LED Screen is enclosed in an enclosure or casing, air-conditions are needed for cooling when the environment temperature reach 30 oC.

- Keep enough space for maintains.

- Prepare for routing power lines and fiber connection. It is doubtless to route power lines and communication cables in two separate conduits.

- Grounding consideration, we recommend grounding resistance must be less than 10 ohms.

- The relations of the screen body size with the distance of installing. The screen body size and the viewing distance should have the suitable proportion relations. The ordinary circumstances viewing distance may take the screen body opposite angle line length 2~20 double, and the effect is better when the viewing distance is in 6~10 doubles.

- The angle of installing: When the screen body installs, should pay attention to the direction of LED element's light. When the screen body installs at highly, should make the screen body to maintain the certain inclination angle, that in order to causes the direction of shines direct to best viewing position.

- When the screen body installed, we should pay attention to the influence of outside electromagnetic radiation to the displays.

  - Cabinet’s information

Following figs show some information of cabinets. User need know these size before installing.

  - Element Assembly

There are five main parts to installation. They are LED cluster module, driver card, receiver card, scan card, power supply.

Following contents will introduce the installation of these elements.

- Driver card

The driver card is fixed on the LED modules. Sometimes it is regarded as one part of LED module. It only need pluged onto the LED cluster card.

Remark: Please note the direction when you plug the driver card onto the LED cluster card. There has a sign ‘’ to denote the direction.

- LED cluster module

The LED cluster module is a single functional unit and need to be fixed on frame. All modules constitute a display wall for display. Installing LED cluster module needs two personnel necessary. One personnel holds the module to frame in front of display. It must ensure the socket and the hole on frame. Then another one tweak screws to the socket for fixed the module at back of frame.

When installing modules must take note of following two points:

  - The louvers on LED cluster modules must be upwards.

  - The module should be firmly seated against the Display.

- Control board and scan board

Scan board is pluged on Control board. There is a plate to fix them. First fixed the plate on the frame by screws, then put the board on plate as the socket to stanchion and tweak screws firmly.

- Power supply

There needs a plate for installing.

First, install power supply on the plate by screws. There are some sockets on back of power supply. Ensure these sockets and holes of plate on one line. Then use screws to fix it. Second, fix the plate on frame by screws.

  - Installation method

The shape and size of LED display are different; the use of them is diverse and the method of mounting is most changeful. There isn’t the unification standard of the mounting. Following we introduced several commonly used method of mounting.

Following we will introduce two installing method:

- Hang installing

This installing method is hanged the Sign on beam or truss. The general installing process is as following steps. It may has some different of the factual operation.

  - Build up the truss installation.
Warning: Be sure that the truss installation complies with the local regulations regarding such installations and that the truss installation will be able to support the complete load of the Moving-Sign.

  - Assemble elements into a whole Sign.

  - Set up stationary rings on the top of cabinet.
The stationary ring is an eyebolt. User only tweaks the stationary ring into the screw on top of cabinet.

  - Lift up the Sign to the desired height.

  - Place a hoist steel cable or chain around the truss installation above the Sign, and through the stationary rings on the truss beam. Use one hoist steel cable or chain per stationary rings.

Following fig shows the view of this installing method.

A

B

C

A: Truss installation    B: Hoist steel cable    C: Stationary rings

- Pillar installing

This installing method is fixed the Sign on two pillar. The general installing process is as following steps. It may has some different of the factual operation.

  - Build up the truss installation.
Warning: Be sure that the truss installation complies with the local regulations regarding such installations and that the truss installation will be able to support the complete load of the LED display.

  - Assemble elements into a whole display.

  - Installing the pillar.
Warning: User must know the size infromation of pillar.

  - Lift up the Sign to the appointed height.

  - Use bolt to fix the Sign on two pillars.

Following fig shows the view of this installing method.

Remark: All the install methods must be examined the backstop at first. And know the weight of whole display and accessory .User must sure the backstop can support the weight of the display while installing display.

The weight of each display is about 180kg.

- Electrical Connection

  - Cable and common connector

The power and signal connections in the Display use many different types of connectors. Take special care when disengaging any connectors so as not to damage the connector, the cable, or the circuit board.

When pulling a connector plug from a jack, do not pull on the wire or cable; pull on the jack itself. Pulling on the wires may damage the connection.

The following information presents some common connectors and cables encountered during display installation and maintenance.

Fig3.2 Termination Block: Usually used for internal power wires to wires of the same type coming into the Display from an external source.

Fig3.3 Copper Tips:  Just used for supplying current from power supply to LED cluster modules.

Fig3.4Waterproof Connectors: Used for external signal and power connection from cabinet to cabinet.

Fig3.1 Cables C1: power cable used for connecting two LED cluster modules.

C3: ribbon cable used for connecting two LED cluster modules.

C3

C1

  - Circuit Diagram whole System

This system has two communication mode: RS232 communication and TCP/IP network communication. Following give the connection of two communication mode.

  - RS232 communication

RS232 is the default communication. It need the control computer must has a RS232 port. The LED display is connected with RS232 port by commnication as following fig.

Fig3.5 RS232 Connection

If there is no enough RS232 port, please use a “USB/RS232 Adapter” instead of the RS232 port. The other part’s connection is as same as RS232 connection. Following fig shows it.

USB Port

DB9

Fig3.6 USB connection

  - TCP/IP Network (LAN) communication

This communication mode need a TCP/IP adapter for each LED disply and a HUB. Following fig shows the connection of this communication.

RJ45 jack

RJ45 plug

Twisted pair cable

TCP/IP adapter DB9

Fig3.7 TCP/IP Network Connection

Remark:

- The cable between HUB and TCP/IP adapter is the standard network cable(twisted-pair).

- The TCP/IP adapter is installed in LED display. Following content will describe it.

  - Signal connection

There are two parts connection in LED display. One is signal cables connection, anther one is power cables connection. This part we will introduce signal cables connection in cabinet.

Following fig shows the signal cables connection in LED display.

Fig3.8 Signal connection of driver card

Driver card

C3 (16pin)

This system has double-face display wall. Above fig shows the signal cable connection of one side wall, and the other side wall’s cable connection is as similar as it.

CST-DEMO-02C includes eight out port, the number of each port is 1, 2, 3, 4, 5, 6, 7, 8 from down to up. Port 1 and 5, 2 and 6, 3 and 7, 4 and 8 output same signal. So double-face display wall will display the same content.

Port is on bottom of cabinet.

Light Sensor

Power inlet

RS232 Connector

To HUB

C4

RS232

C2

C5

C7

To back side modules

C6

J3

JP1

J2

CST-LT-CB-01

JP6

CST-HD-01M

JP3

JP8

TCP/IP adapter

RJ45

C9

To front side modules

DEMO-02C

C8

Fig3.9 Signal connection of system

Remark:

Cable C5 is connected for RS232 communication, Cable C6 is connected for TCP/IP network communication. And the two cable can’t connected at the same time. If you want to use RS232 communication mode, please connect cable C5. If you want to use TCP/IP network communication mode, please connect cable C6.

Cable and its meaning:

| Cable NO. | Description |
| --- | --- |
| C1 | Power cable: made of 4 cables, used for connecting two adjoining LED modules. |
| C2 | Light sensor cable, made of 5 cables. Connect with DEMO-02C. |
| C3 | Signal cable: Ribbon cable (16pin), used for connecting two adjoining LED modules. |
| C4 | Light sensor cable, made of 5 cables. Connect with light sensor. |
| C5 | Signal cable(RS232): Ribbon cable (10pin), used for RS232 connection. |
| C6 | Signal cable: Ribbon cable (10pin), used for connecting to TCP/IP adapter. |
| C7 | Communication cable: twisted-pair, RJ45 connector, connect to HUB. |
| C8 | Signal cable: Ribbon cable (16pin), connect scan board and front side modules. |
| C9 | Signal cable: Ribbon cable (16pin), connect scan board and back side modules. |

Following fig shows the connection picture.

C5

C6

C8

C9

C2

C4

C7

TCP/IP adapter

JP1

C2

C4

J2

CST-LT-CB-01

J3

  - Power Connection

Power Inlet

Power Supply

L N G

L N G

Power inlet

Surge Protective

Device

Breaker

To power supply

Termination Block

Side front

Side back

Power inlet guide:

Power supply connection:

Fig3.10 Power supplies in Cabinet

Plug into

Bottom of cabinet

Plug

Electrical outlet on wall

Fig3.11 Power connection of outside

Power source for this display is 110VAC/50Hz tri-phase power source. There are 8 pieces power supplies to supply power for LED cluster modules and necessary cards in each cabinet, and one power supply supplys power for many piece modules. All power supply is installed on the back of the frame. And each side includes 4 piece power supplies.

Port information:

Power inlet

RS232 Connector

To HUB Port

Light sensor

  - Fans Connection

The back door of cabinet has designed a space for install fan. User only needs peg the fan on that space by bolt. Then connect it with a Temperature controller(KSD301) by power cable. The Temperature controller is peg on the plate that has installed power supply. There needs connect between the power supply and temperature controller. Following fig shows the connection and its diagram.

Fig3.12 Fans connection

Fig3.13 Fans connection diagram

- Maintenance &Troubleshooting

Important Notes:

- Power must be turned off before any repair or maintenance work is done on the Display.

- Qualified service personnel must make any access to internal Display electronics.

- Study foregoing sections to understand configuration of the Display and the routing flow of power and signal.

  - Maintenance

A yearly inspection should be completed to maintain safe and dependable display operation. This inspection should address the following issues:

- Loose hardware Verify fasteners, such as bolts and rivets, have not come loose. Fasteners should be checked and tightened or replaced as required.

- Excessive Dust Buildup Occasionally it may be necessary to vacuum the inside of the Display cabinet to remove dust/dirt buildup that may interface with airflow.

- Water Intrusion Water can enter the Display where weather seal has come loose or deteriorated or where fasteners has come loose allowing gaps in the panels or where moisture may be entering around hardware. Be sure to check around the lift eyes and bolts to ensure that water has not entered there. If so, replace hardware immediately to prevent more water from entering the Display. Also, check electronic components for possible corrosion.

- Corrosion Check the paint, and look for possible corrosion especially at footings, structure tie points and ground rods.

  - Troubleshooting

The below sub-section contains some symptoms that may be encountered in the displays. This list does not include every possible symptom, but does represent common situations that may occur.

Table: Common symptom remedy

| Symptom/Condition | Possible Cause/Remedy |
| --- | --- |
| One or more LEDs on a single LED cluster module fail to light or fail to turn off. | - Check/replace the driver card. - Check/replace the module. |
| Two rows (included in one row cabinets) that extend from left to the right of the Display are not working. | - Check/replace the +5VDC power supply installed in the cabinet. - Check the power wire connection. - Check/replace Scan board installed in the first right cabinet of this row from back side. - Verify proper use of the software. |
| One or two neighbor rows of LED cluster module do not work or garbled. | - Check/replace power supply/power wires connected to the end modules that are not working. - Replace the end module that is not working on the right side. - Replace the adjoining module that is on the right side of the end module (may it does not export data). - Replace the input ribbon cable connect to the end module on the right. |
| Entire Display fails to work.. | - Check the power cables that entrance into the Display. |
| Display is stuck on bright or dim. | - Check/replace light sensor cable. - Check/replace light sensor/light sensor board. |
| One or few rows of Display do not update information. | - Check internal RS232 cable. - Replace CPU board. |
| Can not send or receive message | - Check if the ring communication path has broken node. |

  - Return and Repair

Parts that are replaced by spare parts can be returned to us for repair. Please enclose your name, address, phone number, and a clear description of symptoms. When getting returned parts, we will inspect, test and repair it and send it back as soon as possible. The repairing work is free for a period of two years from the date of shipment. Each will pay the transportation charges. This means, user will pay charges for transporting goods to us and we will pay charges for return.

We retain the right to refuse part that has been damaged due to the acts of nature or causes other than normal wear and tear.

If you have any other question to ask or need any other service, feel free to contact with us.

- Appendix

A. Some things referenced within the manual

Software

COLOR-LED

Manual

Manual COLOR-LED

Circuit Diagram

Fig3.5 RS232 connection

Fig3.6 USB connection

Fig3.7 TCP/IP network connection

Fig3.8 Signal connection of driver card

Fig3.9 Signal connection of system

Fig3.10 Power supplies in Cabinet

Fig3.13 Fans connection diagram

B. File Log

| File No. | Descriptions |
| --- | --- |

- Statement

User must read the manual before use our product.

Besides lists fitting along with the product disposition, the content of this manual contains doesn’t represent pledge of the company.

This manual may contain technical inaccuracies or typographical errors. Changes are periodically made to the information in this manual; these changes are incorporated in new editions of this manual.

All rights reserved. Our company retains the authority that we can revision related documents of picture sign series product at any time and not beforehand informs.