QUICK STEP GUIDE FOR THE COLOR_LED

Introduction:

The Color_LED software allows you to create and manage a rotation program and transfer the rotation program to the display board.  A rotation program consists of one or many created files such as time, temperature, text, graphics and animation.  Here, we have put together a quick step guide that will allow you to create, save preview and transmit a simple message to the display board.  For a full understanding please read over the complete operating manual.  Once we have successfully installed the software you may begin.

Step 1:  On your desktop you will find an icon called Color_LED.  Double click on the icon and the software will open.

Step 2:  Once the software is open, click once where it says New.  A new window will open.  Click on the field under file and type test.

Step 3:  From the selection on the left, click once on ZRT File and it will highlight.  Now click on ok. Two windows will open.  The first will have your file name, this is your work area.  The second is your zoom window.  This allows you to see exactly what you are typing.  You can move the zoom window so that it is not blocking anything.

Step 4:  Click once on the small black area in the middle of the screen.   This is your edit window.  You will see a cursor in both the small box and your zoom window.  Here is where we will be typing our message.  Before we type a message, we have to select a font.

Click on the font icon    and a new window will appear.  Select the font terminal and the size 12 then click ok and it will bring you back to where we were.

Step 5:  Type WELCOME  in the edit window.  You should also see it in the zoom window.  Once you have done this you can click on the save icon   then close the window by clicking on the x .

Step 6:  We have now created our text file.  The next step is to create the rotation program.  We do this by clicking New again and the window will open.  Click on the field under file and type rotation.  Then from the selection on the left, click once on PROGRAM FILE, and then click ok.  A new window will appear.

Step 7:   You will notice a small black screen in the middle.  Click once on Add and you will notice the small black screen turn red.  Move the mouse on the red screen and right click.  A menu will appear.  Select Add Text.  A window will open and you will see the text file we created called test.ZRT ,  double click on the file name and it will bring you back.  Now you will notice a red line of text at the bottom.

The headings to each column have a meaning.

Type means the type of file that you are adding to the rotation program.

Attribute 1 is where the file is saved.

Attribute 2 is the effect in which the text will appear on the display board.

Attribute 3 is the effect in which the text will disappear on the display board.

Attribute 4 is the speed of the effect 1 being fastest 8 being slowest.

Attribute 5 is the amount of time the text remains on the display board.

- When clicking on the effect name under attribute 2 and/or  3, a menu will appear and this will allow you to change the effect. *N.B.* Null can not be selected under attribute 2.  If it is selected the display board will remain blank.

- When clicking on the speed under attribute 4 a menu will appear and this will allow you to change the speed

- When clicking on the pause under attribute 5 a field will open where you can change the pause.  The pause should be set to 10 which equals 1 second minimum.

Step 8:  Once we have completed the first 7 steps, now we are ready to preview the message.  By clicking on Pre a window will appear on the top left corner of the screen and you will be able to see the preview of what we have done.  Once finished, you can click on quit.

Step 9: We are now ready to send the rotation program to the display board.  From the top toolbar we click once on Send. A menu will appear and then we click once on Send File.  A new window will open where we will see our file rotation.ZPF  Double click on the file name and a window will appear showing that the message is being sent.

You have now successfully sent your first message to your display board.

Please feel free to contact our support office @ 905-660-6888 should you have any further questions.  We will be happy to assist you.

When using the display board as a 10 line unit, the following instructions apply.

Step 1:  On your desktop you will find an icon called Color_LED.  Double click on the icon and the software will open.

Step 2:  Once the software is open, click once where it says New.  A new window will open.  Click on the field under file and type multi.

Step 3:  From the selection on the left, click once on ZText File and it will

highlighted.  Now click on ok. A new window will open.  The default font is already

selected for using 10 lines which is ZFONT 6.  The font can be changed according to

your lines.  You will see a flashing cursor in a long black box.  Here is where we begin to

type our message.

Step 4:  Type HELLO AND.  Now we will hit the enter key on the keyboard.  We will notice the cursor on the bottom and the black box got bigger.  Now you are on the second line.  Type WELCOME.  Everytime we wish to go to next line we just hit the enter key.

For now we will just use 2 lines.

Step 5:   Once you have done this you can click on the save icon   then close the window by clicking on the x.

Step 6:  We have now created our multiline text file.  The next step is to create the rotation program.  We do this by clicking New again and the window will open.  Click on the field under file and type rotation2.  Then from the selection on the left, click once on PROGRAM FILE, and then click ok.  A new window will appear.

Step 7:   You will notice a small black screen in the middle.  Click once on Add and you will notice the small black screen turn red.  Move the mouse on the red screen and right click.  A menu will appear.  Select Add Text.  A window will open and you will see the text file we created called multi.ztx ,  double click on the file name and it will bring you back.  Now you will notice a red line of text at the bottom.

The headings to each column have a meaning.

Type means the type of file that you are adding to the rotation program.

Attribute 1 is where the file is saved.

Attribute 2 is the effect in which the text will appear on the display board.

Attribute 3 is the effect in which the text will disappear on the display board.

Attribute 4 is the speed of the effect 1 being fastest 8 being slowest.

Attribute 5 is the amount of time the text remains on the display board.

- When clicking on the effect name under attribute 2 and/or  3, a menu will appear and this will allow you to change the effect. *N.B.* Null can not be selected under attribute 2.  If it is selected the display board will remain blank.

- When clicking on the speed under attribute 4 a menu will appear and this will allow you to change the speed

- When clicking on the pause under attribute 5 a field will open where you can change the pause.  The pause should be set to 100 which equals 1 second minimum.

Step 8:  Once we have completed the first 7 steps, now we are ready to preview the message.  By clicking on Pre a window will appear on the top left corner of the screen and you will be able to see the preview of what we have done.  Once finished, you can click on quit.

Step 9: We are now ready to send the rotation program to the display board.  From the top toolbar we click once on Send. A menu will appear and then we click once on Send File.  A new window will open where we will see our file rotation.ZPF  Double click on the file name and a window will appear showing that the message is being sent.