MANUAL

LED ELECTRONIC DISPLAY

USER MANUAL OF OUTDOOR PICTURE SIGN

PICTURE SIGN

User Manual of Picture Sign

Outline

The Picture Sign is one of our late developed products. Different from the Video Display that is always connected with a PC, the Picture Sign works asynchronously with the control PC.

With powerful embedded control system and embedded operation software, it can display not only texts, but also colorful pictures, short video (avi) files. All displayed content are stored in on-board memory and can be updated through TCP/IP (LAN). It is controlled by this powerful PC software “Colour_led”.

Colour_led is powerful windows-based and easy-to-use, and allows the creation, scheduling and display of content on the Picture Sign. The software also gives the user the flexibility to control content in an instant.

This software is designed for Windows(R) 9x/NT/2000/XP/Vista/Win7.

Features

- Max. Resolution: 65536pixels.

For example, 256(H)×256(W), 128(H)×512(W).

- Memory Size: SD Card up to 2GB.

- Fonts: all fonts in Windows(R) system.

- Color Shades: Monochrome, RG, or RGB. Each color and each pixel can contain 256 shades.

- Standard Interface: RJ45, RS232.

- Connection Type: TCP/IP(LAN), RS232, Modem, GSM Modem.

- Text editor: Included powerful text editor ZRT Edit. File format is "*.ZRT".

- Support Video: 25 fs@256 × 256 AVI video.

- Pictures: bmp, jpg, png.

- Date & Time Format: Six date format, Four time format.

- Temperature: Celsius, Fahrenheit.

- Move Methods: 45 move in methods, 45 move out methods, and it may set as no move out (Move out is 'NULL').

- Pause Time: User-defined (0~86399 seconds).

- Support Schedule file.

- Setup IP: easily setup sign's IP address, subnet mask and default gateway through network (LAN).

- Setup address & Baudrate for Sign: easily change sign's address and baudrate through RS232.

- Adjust time: System time adjustment.

- Support background and foreground display. For example, background may be avi video or picture, and foreground may be text.

- System Upgrade: easily update firmware through network (LAN) or RS232 port.

- Network: Support many LED signs work in a network.

- Two display mode: Support RGB 32 mode and Grayscale mode.

- Thank you very much that you purchase our product

After opens the packing box, please first inspects the fitting according to the detailed list of supply items. If there are something has damaged or lack, please contact your dealer as soon as possible!

•CATALOG

Foreword	6

Section 1: Safety	7

A: Safety Guide	7

B: Important Safety Precaution	8

C: Important Warning	9

Section 2: Introduction	10

2.1 System Outline	10

2.2 Technical Specification	10

2.3 Element Identification	11

Section 3: Mechanical Installation	13

3.1 Overview	13

3.2 Cabinet information	14

3.3 Common mounting method	14

3.4 Wall mounting method	17

Section 4: Electrical Connection	19

4.1 Cable and common connector	19

4.2 Circuit Diagram whole System	20

4.3 Signal connection	21

4.4 Power cable connection	25

4.4.1 External power cable connection	25

4.4.2 Internal power cable connection	25

4.5 Fan connection	28

Section 5: Maintenance	29

5.1 Maintenance	29

5.2 Cleaning Sign	30

5.3 Replacement	30

5.4 Troubleshooting	32

5.4.1 Card information	32

5.4.2 Troubleshooting	34

5.5 Return and Repair	35

Section 6: Appendix	36

Appendix A: Jumper for cards	36

Appendix B: Some cables define	39

Appendix C: File log	41

Section 7: Statement	42

# Foreword

The foreword partial has mainly outlined the user manual of picture sign. Mainly includes:

- About this Manual

- Information feedback

About This Manual

This manual is about picture sign. It explains the electronic connection, installation, maintenance and troubleshooting of the sign.

Reader of the Manual

This manual is for the user and the maintenance of equipment personnel as well as who needs to purchase our product to write. Hope it will bring some help for you!

Contents of the manual

This manual includes seven parts: Safety, Introduction, Mechanical Installation, Electrical Connection, Maintenance, Appendix and Statement.

- Safety: Gives some safety indication.

- Introduction: Describes the basic information needed to use the system of picture sign. Take time to read the entire introduction as it defines terms and explains concepts used through the manual.

- Mechanical Installation: Gives guidance on sign assembling and mounting.

- Electrical Connection: Gives guidance on terminating power and signal cable of the sign.

- Maintenance: Describes such things as replacing basic picture sign components or modules, troubleshooting the sign, performing general maintenance.

- Appendix: Lists the software, manuals referenced within this manual.

- Statement: States the copyright and others.

# Section 1: Safety

A: Safety Guide

Personal protection

- Ensure you understand and follow all the safety guidelines, safety instructions, warnings and notes mentioned in this manual.

- Wear a hard hat to reduce the risk of personal injury.

- Be careful while working with heavy loads.

- Mind your fingers while working with heavy loads.

Installation personnel

The installation must be performed by authorized and qualified technical personnel only.

Accredited safety officers must ensure the safety of the site, construction, assembly, connection, use, dismantling, transport etc. of such safety critical systems.

Notes

Installation should be performed only after you are thoroughly familiar with all of the proper safety checks and installation instructions.

Do not modify and/or replace any component. We use specific materials and manufacturing processes in order to achieve part strength. Consult us for assistance with custom applications.

Always follow our installation manual. Contact us if you should have any question regarding the safety of an application.

Product care

Structural and mounting components should be kept dry, clean and otherwise maintained in a manner consistent with part design. Our products must be used in a manner consistent with their design and inspected on a routine basis for security, wear, deformation, corrosion and any other circumstances that may affect the load handling capability of the part.

This product should be operated from the type of power source indicated on the marking label. If you are not sure of power available, consult your dealer or local power company.

We recommends inspections at regular intervals for all installations and increasing in frequency for more critical installations. If a part is found to have damage, which may cause a decrease in load capability, the part must be removed for service or replaced immediately.

B: Important Safety Precaution

- Read and understand these instructions.

- Heed all warnings.

- Follow all instructions.

- Personnel must be professional to installing and maintenance.

- Unplug this product from the electrical outlet before cleaning. Clean only with materials or chemicals that are inert, nonabrasive, noncorrosive and non-marking. Consult the manufacturer for further advice should any doubts exist regarding any cleaning procedure.

- Do not block ventilation openings.

- Avoid locations that are subject to direct sunlight (except outdoor models), excessive heat, moisture and dust such as radiators, heat registers, stoves, or other apparatus (including amplifiers) that produce heat.

- Don’t defeat the safety purpose of the polarized or grounding type plugs/sockets. If the provided sockets/plugs are damaged then replacement of the defective parts must be undertaken immediately.

- Protect the power/data cables from being walked on or pinched particularly at plugs, convenience receptacles, and the point where they exit from the apparatus. Replace damaged power/data cables immediately.

- Only use accessories specified by the manufacturer.

- Disconnect the power to this apparatus during lightning storms or provide suitable additional lightning protection. Unplug this apparatus when unused for long period of time.

- Refer all servicing to qualified service technicians/personnel. Servicing is required when the apparatus has been damaged in any way, such as power-supply cable or plug is damaged, the apparatus does not working normally, or has been dropped.

- Use only with systems or peripherals specified by the manufacturer. Use caution during lifting/moving or transporting to avoid damage by possible tipping.

- Please check truss installation when you mounting the picture sign.

- Must know the weight for this system before mounting.

C: Important Warning

Important warning:

- Danger of electric shock
Don’t open or touch! To reduce the danger of electric shock, don’t remove cover.

The lightning flash with an arrowhead is intended to tell the user that parts inside the product may cause a danger of electrical shock to persons.

- Danger of fire
To protect against danger of fire by overloading of power cables. Power cable may be connected in parallel. Each power source cable supplying for picture sign should be protected by a circuit breaker or fuses rated.

- Working environment
Temperature: -20℃ to +60℃; Humidity: 15%~95% RH.

- Electro-Static protect
LED components used in picture sign are Electro-Static discharge sensitive. To prevent the possibility of destroying LED components do not touch either in operation or while switched off. Servicing or maintaining must be with anti-static instrument (such as ESD hand/heel straps).

- The equipment must be earthed
In order to protect against danger of electric shock, the installation should be properly grounded. We recommend a resistance to ground less than 10 ohms.

- Power cables
The power cables delivered with this system have special properties for safety. They are not user serviceable. If the power cables are damaged, replace only with new ones. Never try to repair a power cable.

- Data cables
The data cables provided with this system have special properties for safety. They are not user serviceable. If the data cables are damaged, replace only with new ones. Never try to repair a data cable.

- Keep flammable materials away from the equipment (such as curtains). A lot of heat emanated from picture sign while working. Proper ventilation must be provided.

# Section 2: Introduction

This part will introduce some elementary knowledge and technical specification of the system. It includes following content:

- System outline

- Technical specification

- Element identification

2.1 System Outline

The system is outdoor picture sign. In structure, we adopt advanced modularized design that allows easy installation and maintenance. It can be indoor/semi-outdoor use with different sizes and pitches.

For manage picture sign, our company has developed a manage software: Colour_LED_II. The software runs on control PC. It is powerful windows-based and easy-to-use, and allows the creation, scheduling and display of content on the Picture Sign. The software also gives the user the flexibility to control content in an instant. The more information please refer to Colour_LED_Help.

The picture sign can communication with control PC by RS232, Modem, GSM modem, or LAN. User can choose the communcation method according as your need.

2.2 Technical Specification

Table1 list the general technical specification of this picture sign.

Table1: Technical Specification

| Item | Technical Parameters |
| --- | --- |
| Pitch | Pitch 16mm |
| Pixel Configuration | 1R1G1B |
| Display Resolution | 48(H)144(W) |
| Brightness | 6500cd/m2 |
| Brightness control | Auto-dimming by sensor |
| View distance | Horizontal: 120°; vertical: 45° |
| LED Driving Mode | Static constant current |
| Power Input | 110V AC, 50Hz |
| Maximum power | 1.4KW |
| PC Communication Mode | TCP/IP |
| Communication distance | 10meters |
| Waterproof level | IP65 Front, IP54 Back |
| Continuous Working Time | Unlimited |
| Lifetime of LEDs | > 100,000 Hours |
| Working Environment | Temperature: -20 oC～+60 oC; Humidity: 15%～95%RH |

2.3 Element Identification

The following illustrations depict some of the more commonly accessed elements of the Picture sign.

- LED Cluster Module: An aggregation of some LED clusters/pixels and driver circuits.

- Control card (CST-PS-CB-01+CST-PS-CORE-2410-02b+SD card)

Receives data from control PC, and transmits them to scan card after processing. Fig2.1 shows it.

- Scan card (CST-VS-SB-02A)

Scan card is used for receiving data from control card, and then transmit to LED modules after processing. Fig2.2 shows it.

- Brightness control card (CST-LT-CB-01)

Receive the environment’s brightness signal and process them. Fig2.3 shows it.

- Light sensor

Used for Automatic Brightness Adjustment. Fig2.4 shows it.

- Temperature sensor

It is used for getting temperature of environment or internal. It is an optional element for system. Fig2.5 shows it.

- Power supply

It is AC/DC adapter. Converts AC line voltage from the load centre to low DC voltage, and provides power for some cards and LED cluster modules. All the power supplies are switching power supply. There are many kinds power supply. Fig2.6 shows a power supply.

[Fig2.2 Scan card]

[Fig2.1 Control card]

[Fig2.4 Light sensor]

Light sensor

[Fig2.3 Brightness control card]

[Fig2.5 Temperature sensor]

[Fig2.6 Power Supply]

# Section 3: Mechanical Installation

3.1 Overview

Because every installation site is unique, there is no standard method of mounting. As manufacture of LED picture sign, we don’t supply support structure, we can only provide recommending for the support structure design. It is customer’s responsibility to ensure that the structure and mounting hardware are able of supporting the Sign, and according with local laws.

Support structure design depends on the mounting methods, sign’s size and its weight. The structure design is critical and should be done only by a qualified individual and organization.

The following must be considered first before installation:

- Design support structure and groundwork; calculate carefully the needed strength according to local codes; especially note the wind load and the issue of Seismic Zone.

- The mounting structure will provide a straight and square-mounting framework for the Sign.

- For the reason of heat dissipation, the waterproof of the back of LED module is IP45; consider if any added shield needed for more protection. If LED Sign is enclosed in an enclosure or casing, air-conditions are needed for cooling when the environment temperature reach 30 oC.

- Keep enough space for maintains.

- Prepare for routing power lines and fiber connection. It is doubtless to route power lines and communication cables in two separate conduits.

- Grounding consideration, we recommend grounding resistance must be less than 10 ohms.

- The relations of the LED sign size with the distance of installing. The LED sign size and the viewing distance should have the suitable proportion relations. The ordinary circumstances viewing distance may take the LED sign opposite angle line length 2~20 double, and the effect is better when the viewing distance is in 6~10 doubles.

- The angle of installing: When the LED sign installs, you should pay attention to the direction of LED element's light. When the LED sign installs at highly, should make the LED sign to maintain the certain inclination angle, that in order to causes the direction of shines direct to best viewing position.

- When the LED sign installed, we should pay attention to the influence of outside electromagnetic radiation to the signs.

3.2 Cabinet information

This system is outdoor picture sign, and it includes one cabinet. A cabinet is also composed of small module elements, such as LED cluster module, power supply etc. Following give the size figure for this cabinet.

[Fig3.1 Cabinet]

3.3 Common mounting method

The shape and size of LED sign are different; the use of them is diverse and the method of mounting is most changeful. There isn’t the unification standard of the mounting. Following we introduced several commonly used method of mounting.

- Hang type

Hang type include two types. One is hanging up at the ceiling or other beam (fig3.2a shows). The LED Sign is hanging up at ceiling by the bracket. Another is hanging at wall (fig3.2b shows). The LED Sign is hanging at wall by the bracket.

- Swing type

Swing type is an especial of hang type. This method is hanging sign at wall by a bracket too. But the bracket have a suspend arm for Swing. Fig3.5 shows it.

- Desktop installation

This method must design a pedestal to support the LED sign. And the LED sign is installed on the pedestal. Fig3.3 shows it.

- Movable structure

This method is base on desktop installation. It is different from desktop installation there are some wheels on the pedestal. So it can move easily. Fig3.4 shows it.

- Insert type

Insert type is a method that the LED sign is inserted in wall or other object. Fig3.6 shows it.

[Fig3.2a Hang type I]

Ceiling

Bracket

LED Sign

Side

LED Sign

Ceiling

Bracket

Face

Wall

LED Sign

Bracket

Bracket

Face

Side

Wall

LED Sign

[Fig3.2b Hang type II]

[Fig3.3 Desktop installation]

Pedestal

LED Sign

Side

Face

LED Sign

Pedestal

[Fig3.4 Movable structure]

LED Sign

Pedestal

Wheel

Face

Side

LED Sign

Pedestal

Wall

Wall

Suspend arm

Close

Side

Open

LED Sign

Wall

Face

[Fig3.5 Swing type]

LED Sign

Wall

[Fig3.6 Insert type]

Above mentioned methods is several commonly used method of mounting. About the small LED sign, we commonly use hang type or swing type. About the large LED sign, we commonly use desktop installation or insert type. If the LED sign needs move frequently should use movable structure. Certainly, there are many other methods of mounting. We impossibility introduce all methods. User can choose an appropriate method of mounting according as your idiographic circs. And you can give your idea about mounting to us.

Warning: All the mounting methods must be examined the backstop at first. And know the weight of whole sign and accessory. User must sure the backstop can support the weight of the LED sign while mounting.

3.4 Wall mounting method

This mounting method used right-angle bracket to hang sign on wall. Fig3.7 shows the bracket. Following we will introduce the general mounting process of this method by an example. It may have some different of the factual operation.

  - Build up the truss installation.
Set up hole that used for fixing bracket (wall part) on wall. Then use bolt fixed bracket onto wall as fig3.10 shows.
Note: User must confirm the distance between each bolt and the height before fixed bracket onto wall.
Warning: Be sure that the truss installation complies with the local regulations regarding such installations and that the truss installation will be able to support the complete load of the LED sign.

  - Installing bracket (cabinet part) on cabinet.
Use bolt fixed the cabinet part brackets onto cabinet as fig3.9 shows.

  - Lift up the sign to the appointed height.

  - Use bolt to fix the two parts bracket in together so that the sign is fixed on wall. Fig3.11 shows it.

[Fig3.7 Right-angle bracket]

For fixed two parts by bolt.

For use bolt to fixed on cabinet

For use bolt to fixed on wall.

For fixed bracket by bolt

Back of Cabinet

[Fig3.8 Back of cabinet]

[Fig3.9 Cabinet part installing]

Bolt

Bolt

Bracket

Sign

Side View

[Fig3.11 Wall mounting]

[Fig3.10 Wall part installing]

Bolt

Bracket

# Section 4: Electrical Connection

4.1 Cable and common connector

The power and signal connections in the picture sign use many different types of connectors. Take special care when disengaging any connectors so as not to damage the connector, the cable, or the circuit board.

When pulling a connector plug from a jack, do not pull on the wire or cable; pull on the jack itself. Pulling on the wires may damage the connection.

The following information presents some common connectors and cables encountered during picture sign installation and maintenance.

RJ45 connector:

Usually used for internal data or signal cable connection input or output for communication. RJ45 jack is installed on PCB card.

Ribbon cable

Connector

Ribbon cable:

Usually used for internal data or signal connection. For example, between LED modules connection is use ribbon cables.

Power cable:

Usually used for internal power cable connection. The power supply connects to cards or LED modules by this cable. It made up of four cables with two colors; one color cable is “VCC”, and another color is “GND”.

Light signal cable:

Usually used for internal light signal connection that the brightness control card to control card or output to light sensor. It made up of five cables: 1-blue, 2-green, 3-white, 4-black, 5-red.

Temperature signal cable:

Usually used for internal temperature signal connection or output to temperature sensor. It is made up of three cables: 1-red, 2-yellow, 3-green.

4.2 Circuit Diagram whole System

The picture sign can connect with PC by RS232, and it can also use modem communication. Besides, it support control by TCP/IP network (LAN) too. This system uses the TCP/IP communication mode. Following content will introduce this communication mode:

[Fig4.1 TCP/IP network connection]

4.3 Signal connection

Data/signal cable connection in picture sign includes LED modules, control card, scan card, light sensor, temperature sensor and so on.

First, we will describe data/signal cable connection between LED modules. Data cable from scan card output port connect to the first module’s input port by ribbon cable, then through output port connect with next LED modules that in one row. Following give the data cable connection figure between LED modules in the cabinet.

[Fig4.2 LED modules connection]

Control card

COM1: Communication port, Use RS232 communication or set parameters.

LAN: Receiver port, Receives data from PC through network cable.

OUT: Data out port, Transmits data to scan card.

JP2: TEMP port, for connecting with temperature sensor.

JP3: Connect with brightness control card.

POWER: Connect to 5V power supply.

LAN

OUT

POWER

JP3

JP2

COM1

Above contents has described data cables connection for LED modules. Following will introduce control system connection in picture sign. At first user must know the information for some cards before introducing the system connection.

Scan card

TX: Transmit data port.

RX: Receive data port.

OUT1~OUT8: data output port. Output signal to LED modules. Sometimes only use some ports of them.

POWER: Connect to 5V power supply.

OUT1

OUT2

OUT3

OUT4

OUT5

OUT6

OUT7

OUT8

Brightness control card

J2: Light sensor port, used for connect to light sensor.

J3: Protect-Temp port.

J4: +5V DC Power port.

JP1: Bri-out port, for brightness signal output.

J4

JP1

J2

J3

The control system connection has some difference with different communication mode and different picture sign. This system uses TCP/IP communication mode. Following give the connection schematic figure for this communication mode.

[Fig4.3 connection schematic figure for master sign]

4.4 Power cable connection

All power comes into LED sign through a power inlet, and connects with power supplies to supply power for LED modules and cards.

Power cable connection includes two parts: outside power cable connection and in LED moving sign power cable connection.

### 4.4.1 External power cable connection

The power cable connection of external is very simply. It needs to have electrical outlet (a Plug AC female) near the picture sign.

We use special designed outdoor sockets for power, data connection. These sockets are designed with waterproof. And there is a special power cable with plug for connecting to power input socket.

User only needs to plug the plug male onto the electrical outlet (plug AC female). The plug male is connected with LED sign by power cable from the power input port on cabinet. The electrical outlet (plug AC female) is from power source. Following fig shows the power cable connection.

[Fig4.4 External power cable connection]

Warning: To protect against risk of fire by overloading of power cables, please know the load of power source cable and the power of sign. And must ensure the power source cable can load the power of LED sign. It is best never connects other high-power equipment with a power source cable while it connects with LED sign.

### 4.4.2 Internal power cable connection

In LED sign, LED modules, control card, scan card and other element need to be provided power by power supply. All power comes into LED sign through power inlet on cabinet. After power input cabinet, it always connects with surge protective device for safe, and then connects to power supplies by power cable (Sometimes it connects to power supply without surge protective device). Power supply provides power for LED modules and cards so that it needs to connect power supply to LED modules or cards by power cable. One power supply may provide power for many LED modules. In general, each row’s cabinets need one route power line.

Following, we will introduce how to connect power cable in LED sign. It may have some difference from fact such as in cable’s color, number of power supply etc.. But their connection is similar to following described.

Power input guide

Power input cabinet includes two states as following figs show.

AC Input

To power supply

No Surge Protective Device

Surge Protective

Device

Breaker

Termination Block

Include Surge Protective Device

Power inlet on cabinet:

Power supply connecting guide

Cable connecting of power supplies may be different with different LED sign. One power supply may provide power for many LED modules and cards.

More than one power supply: This LED sign needs more than one power supply to provide power. In this status, all power supplies are connected in series. Following fig is a schematic diagram of cable connecting of power supply.

Remark: The color of cable may be different from fact.

Note: Power source for this system is 110V AC/50Hz tri-phase power source, and it includes seven pieces power supplies. The color of cable may be different from above figure, but the cable connection is similar to it. User can refer to above power supply connection figure for power cable connection.

4.5 Fan connection

The back door of cabinet has designed a space for installing fan. User only needs to fix the fan on that space by bolt. Then connect it with a fan controller by power cable. Following fig shows the connection diagram. The power cable color may be different from fact, but the power cable connection is similar to it.

[Fig4.5 Fans connection diagram]

Fan controller (CST-FC-SCR-03) --- Fan controller controls fans turn on/off. When temperature is more than a defined temperature(45oC), fans will work automatically.

# Section 5: Maintenance

Important Notes:

- Power must be turned off before any repair or maintenance work is done on the picture sign.

- Qualified service personnel must make any access to internal sign electronics.

- Study foregoing sections to understand configuration of the sign and the routing flow of power and signal.

5.1 Maintenance

A yearly inspection should be completed to maintain safe and dependable picture sign operation. This inspection should address the following issues:

- Loose hardware Verify fasteners, such as bolts and rivets, have not come loose. Fasteners should be checked and tightened or replaced as required.

- Excessive Dust Buildup Occasionally it may be necessary to vacuum the inside of the Picture sign cabinet to remove dust/dirt buildup that may interface with airflow.

- Water Intrusion Water can enter the Picture sign where weather seal has come loose or deteriorated or where fasteners has come loose allowing gaps in the panels or where moisture may be entering around hardware. Be sure to check around the lift eyes and bolts to ensure that water has not entered there. If so, replace hardware immediately to prevent more water from entering the Picture sign. Also, check electronic components for possible corrosion.

- Corrosion Check the paint, and look for possible corrosion especially at footings, structure tie points and ground rods.

- Cables Check power cables and signal/data cable of system. If power or data cables are damaged, replace only with new ones.

- Fans Failure Every fan installed on the backdoor of cabinet is controlled by fan controller and is turned on when the surface temperature of the cabinet reaches about 45oC. Fans should be checked more often if the LED sign is located in a dusty environment. Fan blades and filters must be kept clean.

- Dustproof cover The dustproof cover with filter is installed on the air inlet of cabinet. In order for avoiding dust blocking the ventilation opening, we suggest user replace the dustproof filter at least once every year. User needs to open the dustproof cover and replace a new filter, then lid the cover.

5.2 Cleaning Sign

Due to outdoor use the sign are exposed to all kinds of weather conditions. Sand, dust, smog and other dirt adhere on the LED sign and because of that the performance of the display is reduced. So we recommend cleaning LED cabinet at regular intervals.

Necessary tools

• Vaporizer with a non aggressive detergent.

• Soft hand brush with long hair.

• Garden hose with a spray nozzle.

• Compressed air.

Cleaning process

- Seal up the data and power sockets of the cabinet with a power and data cable. Make sure that all plug holder clamps are locked firmly.

- Ensure that the unused output ports of cabinet are sealed with a dummy plug.

- Vaporize, through different directions, the non aggressive detergent on the shaders and LED’s.

Warning: Don’t use industrial grease removers. Use only materials or chemicals that are inert, nonabrasive, noncorrosive and non-marking.

- Brush down all dirt of the LED’s and the shaders using a soft hand brush.

Warning: Don’t use a hard bristled brush.

- Wash away the remaining soap with plenty of fresh water.

Warning: Don’t submerge the sign fully or partly in water or other liquids.

- Repeat from step 3 until the cabinet is clean.

- Blow the surface dry with compressed air.

5.3 Replacement

Replacing LED modules

- Read follow the safety instruction in section 1.

- Disconnect the power and data cables and all the cable strings from LED modules.

- Loose these screws that fixed LED modules on grid of cabinet.

- Pushing the LED modules forward out of the grid.

Warning: Ensure you have a good grip on the LED module while releasing the module. We recommend there has one people hold the module in front at the same time while pushing.

- Hand over the released LED module in front of the picture sign.

Note: It’s possible to pull back the LED module through the grid opening if required.

- Snap in a new LED module from the front of the sign.

Warning: Ensure to orient the LED module correctly while snapping in. The louvers on LED cluster modules must be upwards. And the weather-stripping on the back edge of the module must be intact and implanted in the slot of plastic module box.

- Use screws fixed new LED modules onto grid.

Warning: The module should be firmly seated against the grid of cabinet.

- Connect the new LED modules with cables.

Warning: Ensure to fasten the captive lock of the plug.

Replacing driver card

Sometimes driver card is regarded as one part of LED module, and it is fixed at rear of LED module by socket. To remove a driver card from module, disconnect cables and wires attached, then plug out the driver card from socket.

When installing a driver card to module, reverse the previous steps.

Replacing control card or scan card

There is no special handwork to replace these cards but loose screws for fixing and disconnect the cables and wires attached, and then you can take out it easily.

When installing a switch card or scan card, reverse the previous steps.

Replacing power supply

The power supply is fixed on a plate by bolt. And the plate is installed on cabinet. Loose these screws that fixed plate on cabinet, then you can take out the plate from cabinet. Now, you only need loose screws for fixing and disconnect the cables and wires attached of power supply, and then you can take out the power supply easily. When installing a new power supply, reverse the previous steps.

5.4 Troubleshooting

### 5.4.1 Card information

For the picture sign, we have a special indication for simple trouble. User can judge where those problems are and find way to solve the problems from the LED indicator on control card and scan card.

[Fig5.1 LED indicator of control card]

Control card

Status of LED indicators information:

LED 1 --- Network connection indicator

ON – Network has connected; OFF – Network is not connected.

LED 2 --- Network communication indicator

Flash – Network is receiving/transmitting data; No flash – Network has no data receiving/transmitting.

LED 3 --- 5V power supply indicator

ON – Normal; OFF – 5V power supply has fault.

LED 4 --- Display refresh indicator

Flash – FPGA is receiving ARM9 display data; No flash – has no display refresh

LED 5 --- FPGA state indicator

Flash – Normal; No flash – FPGA no work normally or 80M Crystal oscillator has fault.

LED 6 --- 3.3V power supply indicator

ON – Normal; OFF – 3.3V power supply has fault.

LED 7 --- ARM9 state indicator

ON – Normal; OFF – ARM9 doesn’t startup normally.

LED 8 --- ARM9 state indicator

ON – Error; OFF – Normal.

LED 9 --- CPLD state indicator

Flash – Normal; No flash – CPLD no work normally.

- Scan card

LED 4

LED 3

LED 2

LED 1

[Fig5.2 LED indicator of scan card]

Status of LED indicators information:

LED 2 --- 5V power supply indicator

ON – Normal; OFF – 5V power supply has fault.

LED 1 --- 3.3V power supply indicator

ON – Normal; OFF – 3.3V power supply has fault.

LED 3 --- Data receiving indicator

Flash – Normal; No flash – not received data.

LED 4 --- Data transmitting output indicator

Flash – Normal; No flash – FPGA no work normally or 80M Crystal oscillator has fault.

### 5.4.2 Troubleshooting

The below sub-section contains some symptoms that may be encountered in the picture signs. This list does not include every possible symptom, but does represent common situations that may occur.

Table 2: Common symptom remedy

| Symptom/Condition | Possible Cause/Remedy |
| --- | --- |
| One or more LEDs on a single LED cluster module fail to light or fail to turn off. | - Check/replace the driver card. - Check/replace the module. |
| Two rows (included in one row cabinets) that extend from left to the right of the Picture sign are not working. | - Check/replace the DC power supply installed in the cabinet. - Check the power wire connection. - Check/replace data card installed in the first right cabinet of this row from back side. - Verify proper use of the software. |
| One or two neighbor rows of LED cluster module do not work or garbled. | - Check/replace power supply/power wires connected to the end modules that are not working. - Replace the end module that is not working on the right side. - Replace the adjoining module that is on the right side of the end module (maybe it does not export data). - Replace the input ribbon cable connected to the end module on the right. |
| Entire sign fails to work. | - Check the power cables that entrance into the Picture sign. |
| Picture sign is stuck on bright or dim. | - Check/replace light sensor cable. - Check/replace light sensor/light sensor card. |
| One or few rows of picture sign do not update information. | - Check internal data cable. - Replace CPU board. |
| Can not send or receive message | - Check if the ring communication path has broken node. |

5.5 Return and Repair

Parts that are replaced by spare parts can be returned to us for repair. Please enclose your name, address, phone number, and a clear description of symptoms. When getting returned parts, we will inspect, test and repair it and send it back as soon as possible. The repairing work is free for a period of two years from the date of shipment. Each will pay the transportation charges. This means, user will pay charges for transporting goods to us and we will pay charges for return.

We retain the right to refuse part that has been damaged due to the acts of nature or causes other than normal wear and tear.

If you have any other question to ask or need any other service, feel free to contact with us.

# Section 6: Appendix

Appendix A: Jumper for cards

Following content will introduce the jumpers setting method and function on control and scan card.

- Jumper for control card(CST-PS-CB-01)

1

3

2

S10

Jumper S10  on control card:

1> If set S10 as , user can set sign’s parameters information by VSConfigure software.

Set parameters steps:

Step1 --- Connect COM port on PC and COM1 port on control card by RS232 cable.

Step2 --- Power on the control card.

Step3 --- Run VSConfigure software on PC, and set sign’s parameters information.

2> If set S10 as , user can use RS232 communication mode between PC and Picture Sign (Connect COM port on PC and COM1 port on control card by RS232 cable).

Now, user needs to operate according as the following steps if want set sign’s parameters by VSConfigure software:

Step1 --- Set S10 as .

Step2 --- Connect COM port on PC and COM1 port on control card by RS232 cable.

Step3 --- Power on the control card.

Step4 --- Run VSConfigure software on PC, and set sign’s parameters information.

Step5 --- After setting parameters, set S10 as , use RS232 communication between PC and Picture Sign.

Note: If use TCP/IP communication mode, please set jumper S10 on control card according as the first method; if use RS232 communication mode, please set jumper S10 on control card according as the second method.

★ Remark: After modifying IP address for Sign, please connect jumper (connect R28) on core card (CST-PS-CORE-2410-02).

- Jumper for scan card(CST-VS-SB-02A)

1> Jumper definition for scan card

S1: Scan card selects parameters where are from

When S1 is ON, scan card reads saved parameters on it (the parameters are saved in AT24C02);

When S1 is OFF, scan card selects the parameters from the previous cascade.

S2: enable setting

When S2 is ON, column enable reverse, it can avoid burning out display card when connect with power.

When S2 is OFF, select “normal”. The default is OFF.

S3: Parameters save

When S3 is ON, save parameter spring, connect S3 about three seconds then disconnect it, the parameter will be saved in scan card;

When S3 is OFF, save parameter finish. The default is OFF.

2> Parameters configuration operation steps

Step1 --- Set control card can use VSConfigure software to set parameters (Set S10 on control card as ).

Step2 --- Connect control card and scan card by data cable, and connect COM port on PC and COM1 port on control card by RS232 cable.

Step3 --- Power on the control card and scan card.

Step4 --- Run VSConfigure software on PC, and set sign’s parameters information.

Step5 --- Connect S3 on scan card about three seconds, and then disconnect it, the parameters are saved in AT24C02 on scan card.

Step6 --- Connect S1 on scan card, scan card reads saved parameters on it.

Note: If the system is double-face picture sign, for scan card in slave sign, user needs to connect it to control card in master sign by data cable and set sign’s parameters (please refer to above parameters configuration operation steps), after complete setting, return to normal system connection.

Appendix B: Some cables define

- Power cable

The power cable used for external or input/output are mad up of three wires. This power cable connector has three pin. Wiring standard for power cable as following:

Pin1 --- L   Pin2 --- N   Pin3 --- GND

- Data cable

The data cable used for external or input/output of LED cabinet are twisted-pair. Twisted-pair cable contains 4 pairs of wires. Each pair consists of a solid (or predominantly) colored wire and a white wire with a stripe of the same color. The pairs are twisted together. To maintain reliability on communication, you should not untwist them any more than necessary (like about 1cm). The connector of this data cable includes many kinds: RJ45, waterproof connector, and DB-9 connector. The wiring standard for this data cable adopt “T-568B” standard for RJ45 plug. Following fig shows this wiring standard.

[Fig6.1 Wiring standard]

Above illustration shown is RJ45 plug, other connector pin also obey this order. Following table give wiring rule for this data cable.

Table3: Wiring rule for data cable (Twisted-pair)

| Connector Pin | Color code | Color |
| --- | --- | --- |
| Pin1 | White orange |  |
| Pin2 | Orange |  |
| Pin3 | White green |  |
| Pin4 | Blue |  |
| Pin5 | White blue |  |
| Pin6 | Green |  |
| Pin7 | White brown |  |
| Pin8 | Brown |  |

The two end connectors of data cable obey above rule. The connector may be RJ45, DB-9 connector, or waterproof connector etc.

- Light signal cable

The light signal cable contains five colored wires: blue, green, white, black, and red. Wiring rule as following:

Pin1 --- Blue wire

Pin2 --- Green wire

Pin3 --- White wire

Pin4 --- Black wire

Pin5 --- Red wire

- Temperature sensor cable

The temperature sensor cable contains three colored wires: yellow, green, and red. Wiring rule as following:

Pin1 --- Red wire --- VCC

Pin2 --- Yellow wire --- GND

Pin3 --- Green wire --- Signal

Appendix C: File log

| File No. | Descriptions |
| --- | --- |

# Section 7: Statement

User must read the manual before use our product.

Besides lists fitting along with the product disposition, the content of this manual contains doesn’t represent pledge of the company. We reserve the right to make changes at any time in order to improve design and to supply the best product possible.

This manual may contain technical inaccuracies or typographical errors. Changes are periodically made to the information in this manual; these changes are incorporated in new editions of this manual.

All rights reserved. Our company retains the authority that we can revision related documents of picture sign series product at any time and not beforehand informs.