CST

Operation Manual

VSConfigure V3.0.3

LED ELECTRONIC DISPLAY          Version: 3.0.3

©All Right Reserved

·CATALOG

Section 1: Preface	2

1.1 System Requirement	2

1.2 Important warning	2

1.3 Install software	3

Section 2: Software introduction	4

2.1 Register OCX control	4

2.2 Communication Setting	6

2.3 Basic Setting	8

2.4 Advanced Setting	10

2.5 Other function	12

2.6 Comments	13

Section 3: Statement	15

# Section 1: Preface

The purpose of this manual is to assist in the installation and operation of the VSConfigure V3.0.3.

1.1 System Requirement

The VSConfigure program has the following minimum system requirements:

- Personal computer running Windows 2000/XP Home/Professional or Vista with the most current updates installed (XP is recommended).

- Personal computer using a Pentium 4 class, 2.0GHz or higher microprocessor.

- 128MB RAM or up.

- CDROM Drive.

- Mouse or other pointing device.

- One VSConfigure program installed.

- 100MB hard disk space or up.

- With display mode of 800×600, 16, 24 or 32 bit colors (1024×768, 32 bit colors is recommended).

- 10M Network

1.2 Important warning

Important warning:

- The computer can’t be locked when running VSConfigure program.

- Can’t set screen saver for computer.

- Can’t set system standby for computer.

- Can’t set system hibernates.

- Set never turn off monitor for computer.

- Set never turn off hard disk.

If you uninstall this software and install it again, and you want to reserve information of old disk files, you should save these files in the same root directory with “VSConfigure.exe” before uninstall the software. After install software again, load these files into the root directory of “VSConfigure.exe”. Then the software has reserved the information for old disk files.

1.3 Install software

If you want to use the software, you need to install the software on your computer firstly. In this section, it will describe installing and configuring the VSConfigure software.

Insert the CD labeled “VSConfigure” into your CD disk driver. Double click the icon of your CD, then copy the file named “VSConfigure” to your computer’s disk. This program doesn’t need to be installed. When you double click mouse left-key on the icon of the software on computer monitor desktop it will run.

# Section 2: Software introduction

2.1 Register OCX control

Before running the software, you must register ocx control (VSConfigure.ocx) to your OS(If you use setup program, you are not necessary to register control because setup program will register it automatically). First, we must enter interface  “command prompt”, the step is: click “start” on your lower left corner on your desktop, click “All Programs” and then click “Accessories” and select “Command Prompt” and the follow interface appears:

Assume the path of your OCX control is: C:\ VSConfigure.ocx, the register method is:

After the prompt “DllRegisterServer in C:\VSConfigure.ocx succeeded” appears, we can open the VSConfigure software.

And if you want to unregister this OCX control, you can input command line: regsvr32 –u C:\ VSConfigure.ocx. (Assume the path of your OCX control is: C:\), then there is a prompt: “DllUnregister in C:\VSConfigure.ocx succeeded”.

There is a more quickly method for you to register OCX control, the method is: click “start” on your lower left corner on your desktop, then click “run”, in the prompted dialog input command line: regsvr32 C:\ VSConfigure.ocx, then there is a prompt like above. Of course, if you want to unregister OCX control, you can input command line: regsvr32 –u C:\ VSConfigure.ocx, then there will aloes be a prompt like above.

2.2 Communication Setting

After running the VSConfigure program, it will appear a friendly interface as following figure shows.

In above figure click the “Communication Setting” button to set the communication parameters and running mode.

- Running mode

VSConfigure V3.0.3 has three kinds of running mode: normal, client and server. They are divided according as communication mode, when VSConfigure program connects with transmitter card directly through COM port, use normal mode; the client and server modes are used for remote control, namely, server port connects with transmitter card directly; and client port communicates with server port through network, it operates the transmitter card indirectly through sending command to server port.

- Address

Address of video transmit card (1-255), it is effective under normal and server modes.

- Com Device

Communication COM port with transmitter card, it is effective under normal and server modes.

- Baudrate

COM device baudrate, it is effective under normal and server modes.

- IP/Domain name

IP address or domain name of server port PC connected with transmitter card directly, it is effective under client mode.

- Port

Communication port of server port PC connected with transmitter card directly, it is effective under client mode.

- Connect server/Start server

It is “Connect server” when selecting client mode, namely, connect server port PC connected with transmitter card directly; it is “Start server” when selecting server mode, namely, create connection between server port and transmitter card and start server, wait client port connecting.

- Disconnect server/Stop server

It is “Disconnect server” when selecting client mode, namely, disconnect server port PC connected with transmitter card directly; it is “Stop server” when selecting server mode, namely, server port stop servicing, disconnect the network communication connection with client port.

- Auto connect when switch to current page

It is effective under normal and client modes. If select the check box, it will connect with destination device when the program running or entering the next property page. (if it is normal mode, the program will connect with transmitter card directly; if it is client mode, the program will connect with server port.)

- Connect To LED

It is effective under normal and client modes. When click this button, if it is normal mode, the program will connect with transmitter card; if it is client mode, the program will connect with server port PC connected with transmitter card directly.

Note:

1. User must select the right running mode according as connection mode, or else the program may not work normally.

2. User must ensure that the communication port is not impropriated, or else fail to connect server port.

3. The static box shows the communication information during the program operation.

2.3 Basic Setting

When click “Basic Setting” button, it will appear the interface as following figure shows.

In above figure, user may set some basic information.

- LED coordinate and size

1. X:

Horizontal coordinate value that transmitter card outputs (0~2047), user can change this value by input directly or by spin button or the slider.

2. Y:

Vertical coordinate value that transmitter card outputs (0~2047), user can change this value by input directly or by spin button or the slider.

3. Width:

Width value that transmitter card outputs (0~2047), user can change this value by input directly or by spin button or the slider.

4. Height:

Height value that transmitter card outputs (0~2047), user can change this value by input directly or by spin button or the slider.

- LED brightness

1. Red, Green, Blue:

Red, green and blue brightness value that transmitter card outputs (0~255), user can change this value by input directly or by spin button or the slider.

2. Change GRB value synchronously:

When selecting this check box, red, green and blue brightness value will be changed at the same time.

- LED common settings

1. Virtual Pixel:

When selecting this check box, the transmitter card will output virtual pixels.

Note: When selecting this check box, the red prompt box will become larger. When need to display virtual pixel, user must enter “Advanced Setting” interface and select “Virtual” under “Scan and display parameters” box.

If select “Overlay pixel” or “Big” under “Scan and display parameters” box, the virtual pixel function is noneffective.

2. Black Screen

When selecting this check box, the transmitter card will output black screen.

3. LED Locked

When selecting this check box, the transmitter card will lock output.

4. LED Reverse

When selecting this check box, the transmitter card will reverse output.

5. Normal, RG Alternate, RB Alternate, GB Alternate

Normal: show normally

RG Alternate: red-green display alternately

RB Alternate: red-blue display alternately

GB Alternate: green-blue display alternately

- LED setting operation

1. Connect To LED:

Click this button, the program will connect with destination device.

2. Read From LED

Click this button, the program will read data from destination device.

3. Send To LED

Click this button, the program will send current parameters information to destination device.

4. Save To FLASH

Click this button, the program will save current parameters information to FLASH memorizer of destination device.

- LED communicating information

Display prompt information during communicating with transmitter card.

2.4 Advanced Setting

Click the “Advanced Setting” button, it will popup a warning dialog box as following figure shows.

Note: When you turn to Advanced Setting property sheet, the above warning dialog will appear. There are some important parameters in the advanced setting property sheet, if you changed this setting, the destination device may not display normally. So only the administrator can access it by input password successfully. If want to change these parameters, you can get password from software provider.

When input the right password and press “OK” button, the following interface will appear.

Following will introduce some information about advanced setting.

- Cabinet size/Receive card output offset/show place adjust (both in pixel)

It includes the following content: set receive card output X offset value (0~2047); set receive card output Y offset value (0~1023); set cabinet width value (4~1024); set cabinet height value (4~256). Note: the value for cabinet width and height must be a multiple of 4, when input box lose focus, the program will keep the integral part automatically. Set LED X offset value (0~7); set LED Y offset value (0~7).

- Scan and display parameters

It includes the following content: transmitter card adopts scan or count mode; static scan, 1/4 scan, 1/8 scan, 1/16 scan; overlay pixel or virtual pixel; synchronously scan or asynchronously scan; row enable or column enable; signal horizontal connect or signal vertical connect; horizontal return or vertical return; 4 pixel return, 8 pixel return or 16 pixel return; 4 row output, 8 row output or 16 row output; whether increase scan; whether receive scan; whether double length; whether data reverse; whether half clock.

- Signal transport direction/LED settings

It includes the following content: set signal horizontal transport direction (right to left or left to right) and vertical transport direction (bottom to top or top to bottom); small mode or big mode; off test or off black; TClk normal or TClk reverse; clock delay (0~7); whether half frame; whether half frequency.

- LED setting operation

It includes the following content: connect to LED; read from LED; send to LED; save to FLASH. The function for each button is same as the Basic Setting property sheet.

- LED communicating information

Display prompt information during communicating with transmitter card.

2.5 Other function

- Language

“Language” catalog for the path of the program includes available language file, the program will list all available language, and user can select the language that you want. If this catalog doesn’t have the language, you can create yourself language file according as language file format in this catalog.

- Open configure file

Open configure file that user has created form disk or saved configure file.

- Save configure file

Click, it will appear the following interface:

It includes two save modes: save parameters by reading from LED FLASH and save parameters with current page settings. The first mode is reading data from FLASH of transmitter card and saving as configure file; the second mode is saving current parameters of program as configure file.

- About

Click this button, it will display data and version information for VSConfigure program.

- Close

Click this button, it will exit the program.

2.6 Comments

1. Please register OCX control (VSConfigure.ocx) according as the register steps in section 2.1 before running the program, or else the program will not run.

2. After running program, enter “Basic Setting” sheet and “Advanced Setting” sheet for the first time, the program will read corresponding parameters from destination device and display it on program interface. Enter “Basic Setting” sheet and “Advanced Setting” sheet again, the program will not read corresponding parameters again. If you open configure file, then enter “Basic Setting” sheet and “Advanced Setting” sheet for the first time, the program will not read corresponding parameters too.

3. When click “Save To FLASH”, saved information may be not the parameters information on interface, because the program may be disconnected with destination device, but user can change each parameter value all the same and the parameter values on interface has been changed, and these parameters are not recorded, so “Save To FLASH” is saving each parameter value when communication is successful.

4. When select “Change RGB value synchronously”, here Red, green and blue value may be different, if the certain brightness value doesn’t reach to 0 or 255, and other brightness values has reached to 0 or 255, then other brightness values are unchangeable.

5. VSConfigure V3.0.3 adds some new functions: support manifold language and you can create yourself same format language files according as language file format; add save configure file and open configure file function, user can save the current configure information as configure file and use it next time; add red color window prompt information, user can know window place and size, and change window place and size through dragging window.

6. When change parameter value through dragging slider, because of program sends parameter value to transmitter card timely, we suggest that user doesn’t drag slider with fast speed, or else the slider may be ineffective.

7. If use Window Vista OS, OCX control register must be under safety mode. It is failed to register OCX control under normal mode.

8. VSConfigure V3.0.3 has two selected languages at present: English and Simplified Chinese, the English is the default language. If you need to use other languages, please put the corresponding edited language file(for example: VSConfig_cn_GBK.ini) under the Language directory, which is in application program directory.

# Section 3: Statement

Dear customers:

Thank you for purchasing this product！

For optimum performance and safety, please read these instructions carefully.

Before operating or adjusting this product, please read these instructions completely. Please keep this manual for future reference.

Changes

We provide this manual ‘as is’ without warranty of any kind, either expressed or implied, including but not limited to the implied warranties or merchantability and fitness for a particular purpose. We reserve the right to make changes at any time in order to improve design and to supply the best product possible.

This manual could contain technical inaccuracies or typographical errors. Changes are periodically made to the information in this manual; these changes are incorporated in new editions of this manual.

Warning

When using our company’s software for Non-our company’s equipment, the reliability general not be responsible.

The figure interface in this manual is for guide user while uses the program. Because all figure are cut figure, so it can’t display the content of interface legible. If user wants to read the legible content of interface, please according as your individuality defined to read the legible interface’s content.

Copyright

All rights reserved. No part of this document may be copied, reproduced or translated. It shall not otherwise be recorded, transmitted or stored in a retrieval system without the prior written consent of Comsight.

If this manual’s content have any discrepancy with user has buy product’s software, Comsight retains the right of finally explain.