Sunitron Inc.

QUICK STEP GUIDE FOR THE MULTI-LINE

Introduction:

The Multiline software allows you to create, manage and transfer data to the display board.  The data consists of created files such as time, temperature, text and graphics.

Here, we have put together a quick step guide that will allow you to create, save preview and transmit a simple message to the display board.  For a full understanding please read over the complete operating manual.  Once we have successfully installed the software you may begin.

Step 1:  On your desktop you will find an icon called Multi-Row.  Double click on the icon and the software will open.

Step 2:  Once the software is open, click once on File on the main toolbar. A pull down menu will appear, then click once where it says New.  A new window will open called moving sign parameter setting.  Here we will fill in the info of your sign. The size is your pixel configuration that can be found on the software disk. The color is if you have a single or double color display. Text file name is where we will name our file.  The default name is MAIN.  We can leave this or change it to another name.

Step 3:  Once we have filled in the information we click once on okay.  Two new windows will appear.  The first on the left is our text editor where we create our messages. You will notice our file name on top of the window.  The second on the right is the property window.  Here is where we select different fonts or styles when creating our message.

Please refer to fig 1.1 on pg 8. for full details.

Step 4:  In the text editor you will see a flashing cursor.  Type WELCOME .  The default font,  effect and speed is already set.  These can be changed at your discretion.

For now we will use the default settings.

Step 5:  We have now created a message and now we can preview it.  Click on View on the main toolbar. A pull down menu will appear, then click on Preview.  A new window will open called Simulate and there you will see your actual sign size and the message running with all the random effects.

Step 6:  Once we have previewed our message we can close simulate window by clicking on the x on the right corner of the simulate window.

Step 7:  We have now created a message and previewed the message.  Now we can send the message to the sign. Click on Update on the main toolbar. A pull down menu will appear, then click on Text file.  A new window will open called Update text file and there you will see a button called update.  Click the update button and a new window will open.  Enter address “1” (1,2 if more the one sign). Click on ok and your sign will update.  When finished click once on the close button.

Step 8:  Once we have completed the first 7 steps, now we are ready to save our message.  By clicking on File on the main toolbar a menu will appear, then click on save.  A new window will open.  The default file name will be already highlighted.  Type welcome and then click the save button.  It will automatically exit and bring you back to the main screen.  Click on File on the main toolbar then click on exit.

You have now successfully sent your first message to your display board.

Please feel free to contact our support office @ 905-660-6888 or

toll free @ 1-877-630-8811should you have any further questions.

We will be happy to assist you.